Potential TERMs

Under Consideration

2003 CLRP & TIP FY 2004 – FY 2009

Revised NOx & VOC Calculations

(Based on 2.1 Travel Demand Model Data & Mobile 6 Emissions Factors)

| ###### CONTENT | Page # |
| --- | --- |
| M-07A Voluntary Employer Parking Cash-Out Subsidy | 4 |
| M-24 Speed Limit Adherence (accelerated) | 7 |
| M-47c Employer Outreach for Private Sector Employers (expanded) | 10 |
| M-93 Improve Pedestrian Facilities Near Rail Stations | 13 |
| M-103 150 Taxicab Replacement Program - CNG (expanded) | 15 |
| M-103a 150 Taxicab Replacement Program - conventional vehicles | 19 |
| M-110 10 Transit Stores in Maryland | 21 |
| M-111 Replace Traffic Signals with Lesser Controls | 29 |
| M-113 6 Kiosks in Maryland | 30 |
| M-123 Employer Outreach for Public Sector Employers | 35 |
| M-132 Vanpool Incentive Program (expanded M-77b) | 38 |
| M-133 Metrorail Feeder Bus Service | 42 |
| M-134 Implement Neighborhood Circulator Buses (10) | 46 |
| M-135 Construction of 5950 Parking Spaces at Metrorail Stations | 49 |
| M-142 100 CNG Buses (Purchase CNG buses in place of new diesel) | 52 |
| M-142a 100 CNG Buses (Replace old diesel buses with CNG buses) | 54 |
| M-142b Opt for CNG for Next Replacement/Expansion Bus Purchase | 56 |
| M-142c Replace 952 Bus Fleet with CNG Operated Buses | 58 |
| M-142d WMATA CNG Bus Replacement (buses>12 years old) | 61 |
| M-143 Real Time Bus Schedule Information | 63 |
| M-146 Purchase 100 WMATA buses (ridership growth) | 70 |
| M-147 Expand Fleet with Hybrid/Electric Buses | 73 |
| M-148 WMATA Bus Information Displays with Maps (2000 cases) | 75 |
| M-149 Automatic Vehicle Locator System | 78 |
| M-150 Enhanced Commuter Services- (HOV Facilities) | 81 |
| M-151 Enhanced Commuter Services-US 1 (Reverse Commute) | 84 |
| M-152 Enhanced Commuter Services- (Rail Relief) | 87 |
| M- 154 Extend Peak Hour Service | 90 |
| M-155 Expand Car sharing Program | 93 |
| M-156 Free bus-to-rail/ rail-to-bus transfers | 96 |
| M-158 Free Bus Service Off-Peak | 99 |
| M-159 W15-590 - Diesel Fuel Additive | 102 |
| M-160 Bose Automobile Anti-Air Pollutant and Energy Conservation System | 104 |
| M-161 Diesel Emulsion Fuel Additive (Non-road or Highway) | 106 |
| M-162 Early Engine Retirement (Pre-88) | 107 |
| M-163 Truck Idling (Truck Stops and Auxiliary Power Unit ) | 108 |
| M-164 International Green Diesel Retrofit | 109 |
| M-165 Bike Stations at Rail Station | 110 |
| M-144 Parking Impact Fees (Revenue Generating TERM) | 113 |
| Other Suggested Measures- Under Development |  |
| Transit Oriented Development |  |
| Reducing Congestion |  |
| Heavy Duty Truck Pollution |  |
| Government Actions (ozone action day similar to snow day) |  |
| M-153 Remove 125 Trash Trucks per/day |  |
| M-166 Monthly Transit Pass |  |

| Potential TERMs TRAVEL DEMAND AND EMISSIONS REDUCTION FROM POTENTIAL TERMS UNDER CONSIDERATION FOR CONFORMITY OF THE 2003 CLRP & FY 2004-2009 TIP | 2005 VT | 2005 VMT | VOC | NOx | Project |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 2005 | 2005 | Category * |
| M-07A Voluntary Employer Parking Cash-Out Subsidy | 30205 | 414345 | 0.2022 | 0.3525 | C |
| M-24 Speed Limit Adherence (accelerated) | NA | NA |  | 1.3145 | TR |
| M-47c Employer Outreach for Private Sector Employers (expanded) | 5680 | 77924 | 0.0380 | 0.0663 | C |
|  |  |  |  | 0.7380 |  |
| M-93 Improve Pedestrian Facilities Near Rail Stations | 4625 | 63449 | 0.0310 | 0.0540 | C |
| M-103 150 Taxicab Replacement Program - CNG (expanded) | NA | NA |  | 0.2030 | SP |
| M-103a 150 Taxicab Replacement Program - conventional vehicles | NA | NA |  | 0.1750 | SP |
| M-110 10 Transit Stores in Maryland | 1173 | 32130 | 0.0127 | 0.0261 | C |
| M-111 Replace Traffic Signals with Lesser Controls | NA | NA |  | 0.0026 | TR |
| M-113 6 Kiosks in Maryland | 14 | 165 | 0.0001 | 0.0001 | C |
| M-123 Employer Outreach for Public Sector Employers | 14622 | 200575 | 0.0979 | 0.1706 | C |
| M-132 Vanpool Incentive Program (expanded M-77b) | 1912 | 112572 | 0.0387 | 0.0889 | C |
| M-133 Metrorail Feeder Bus Service | 0 | 6131 | 0.0018 | 0.0047 | C |
| M-134 Implement Neighborhood Circulator Buses (10) | 3214 | 42562 | 0.0211 | 0.0363 | C |
| M-135 Construction of 5950 Parking Spaces at Metrorail Stations | 0 | 112567 | 0.0337 | 0.0868 | C |
| M-142 100 CNG Buses (Purchase CNG buses in place of new diesel) | NA | NA |  | 0.1358 | SP |
| M-142a 100 CNG Buses (Replace old diesel buses with CNG buses) | NA | NA |  | 0.1360 | SP |
| M-142b Opt for CNG for Next Replacement/Expansion Bus Purchase | NA | NA |  | 0.4300 | SP |
| M-142c Replace 952 Bus Fleet with CNG Operated Buses | NA | NA |  | 1.9800 | SP |
| M-142d WMATA CNG Bus Replacement (buses>12 years old) | NA | NA |  | 2.1020 | SP |
| M-143 Real Time Bus Schedule Information | 1273 | 17471 | 0.0085 | 0.0149 | C |
|  |  |  |  | 0.0154 |  |
| M-146 Purchase 100 WMATA buses (ridership growth) | 10540 | 144590 | 0.0706 | 0.1230 | C |
| M-147 Expand Fleet with Hybrid/Electric Buses | NA | NA |  | 0.0600 | SP |
| M-148 WMATA Bus Information Displays with Maps (2000 cases) | 2284 | 31328 | 0.0153 | 0.0266 | C |
| M-149 Automatic Vehicle Locator System | 6588 | 90369 | 0.0441 | 0.0769 | C |
| M-150 Enhanced Commuter Services- (HOV Facilities) | 0 | 50172 | 0.0150 | 0.0387 | C |
| M-151 Enhanced Commuter Services-US 1 (Reverse Commute) | 2089 | 50172 | 0.0204 | 0.0410 | C |
| M-152 Enhanced Commuter Services- (Rail Relief) | 0 | 69282 | 0.0207 | 0.0534 | C |
| M- 154 Extend Peak Hour Service | 5795 | 79495 | 0.0388 | 0.0676 | C |
| M-155 Expand Car sharing Program | 0 | 4261 | 0.0013 | 0.0033 | C |
| M-156 Free bus-to-rail/ rail-to-bus transfers | 5270 | 72295 | 0.0353 | 0.0615 | C |
| M-158 Free Bus Service Off-Peak | 4427 | 60728 | 0.0296 | 0.0517 | C |
| M-159 W15-590 - Diesel Fuel Additive | NA | NA |  | 0.1330 | H |
| M-160 Bose Automobile Anti-Air Pollutant and Energy Conservation System | NA | NA |  | 0.6100 | H |
| M-161 Diesel Emulsion Fuel Additive (Non-road or Highway) | NA | NA |  | 0.1800 | H |
| M-162 Early Engine Retirement (Pre-88) | NA | NA |  | 0.9000 | H |
| M-163 Truck Idling (Truck Stops and Auxiliary Power Unit ) | NA | NA |  | 0.4000 | H |
| M-164 International Green Diesel Retrofit | NA | NA |  | 0.1400 | H |
| M-165 Bike Stations at Rail Station | 115 | 1573 | 0.0008 | 0.0013 | C |
| M-144 Parking Impact Fees (Revenue Generating TERM) | 126694 | 1737963 | 0.8481 | 1.4784 | C |
| Other Suggested Measures- Under Development |  |  |  |  |  |
| Transit Oriented Development | NA | NA | ####### | 0.0000 | C |
| Reducing Congestion | NA | NA | ####### | 0.0000 | TR |
| Heavy Duty Truck Pollution | NA | NA | ####### | 0.0000 | H |
| Government Actions (ozone action day similar to snow day) | NA | NA | ####### | 0.0000 | C |
| M-153 Remove 125 Trash Trucks per/day note 5 | NA | NA | ####### | 0.4860 | H |
| M-166 Monthly Transit Pass | NA | NA | ####### | 0.0000 | C |
| Project Category: TR - Traffic Stream, C - Commute, H - Heavy Duty Vehicles (Engine Technology), SP- Specific Vehicle Type |  |  |  |  |  |

| ## M – 07A Voluntary Employer Parking Cash-Out Subsidy |  |
| --- | --- |
| Description: | Many employers in the region choose to subsidize automobile parking for their employees. These subsidies are executed through payment of parking fees, or the provision of parking space leased or owned by the employer. This TERM would create a program for giving equal compensation “cash-out” to employees who choose not to use the free parking provided by the employer and use alternative modes of travel such as walking, bicycle, transit, ridesharing, etc. |
| Analysis Tool: | Sketch Planning, COMSIS TDM Model (1993 analysis studied only the impact of transit and ridership.) |

Assumptions:

- This TERM will only affect work-related trips.

- The existing parking subsidies offered by each employer can be quantified.

- Participation in this TERM by employers in voluntary.  It was assumed 20% of the regions employers would participate in the program.

- The results are based on a 1993 analysis of mandatory employer parking cash-out subsidy for transit/HOV.

- Trip reduction rate – 4.3%

Summary Impact:

#### Scenario 1: 20 % employee participation

| Daily VT Reduction: | 28,198 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 408,870 | VMT |
| Daily NOx Reductions: | 0.4277 | tons/day |

#### Scenario 2: 10% employee participation

| Daily VT Reduction: | 14,100 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 204,435 | VMT |
| Daily NOx Reductions: | 0.214 | tons/day |

Emission Analysis:

Base 2005 HBW single occupant trips – 3,278,831

Trip reduction rate – 4.3%

Employee participation rate – 20%

Total VT reduction  - 3278831 x 4.3% x 20% = 28,198

Total VMT reduction – 28,198 x 14.5  =  408,870

Daily NOx Emission Reductions:

| Cold Start | 28198 | x | 0.912 grs | x | 1 ton | = | 0.0283 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 408,870 | x | 0.980 grs | x | 1 ton | = | 0.4417 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.4700 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.4277 | tons |

Cost Effectiveness:

The cost of administering the program is estimated at $200,000/year. The employer cost is not estimated.

| Cost Effectiveness NOx = | 200,000 | = $1500/ton |
| --- | --- | --- |
|  | 312 x 0.427 |  |

Alternative Scenario with 10% employee participation in the Voluntary Employer Parking Cash-Out Subsidy program.

The following figures demonstrate the impact on VT, VMT, NOx , and Cost effectiveness if the only 10% of the employees will have to pay for parking throughout the region.

Base 2005 HBW single occupant trips – 3,278,831

Trip reduction rate – 4.3%

Employee participation rate – 10%

Total VT reduction  - 3278831 x 4.3% x 10% = 14,100

Total VMT reduction – 14,100 x 14.5  =  204,435

Daily NOx reductions:  0.214 tons/day

The cost of administering the program: $200,000/year

Cost effectiveness: $3000 /tons

M – 07A Voluntary Employer Parking Cash-Out Subsidy

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 Emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 28198 |  | 30205 |  |  |  |  |  |  |
| VMT | 408870 |  | 414345 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 30205 | x | 0.9905 grs | x | 1 ton | = | 0.03298 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 414,345 | x | 0.6995 grs | x | 1 ton | = | 0.31949 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.35247 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start + Hot Soak | 30205 | x | 2.3454 grs | x | 1 ton | = | 0.07809 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 414,345 | x | 0.2717 grs | x | 1 ton | = | 0.12410 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.20219 | tons |  |
| Cost effectiveness | $200,000 |  |  | = | 1819 | $/ton |  |  |  |
| NOx | 312 | x | 0.35247 |  |  |  |  |  |  |
| Cost effectiveness | $200,000 |  |  | = | 3170 | $/ton |  |  |  |
| VOC | 312 | x | 0.20219 |  |  |  |  |  |  |

| ## M – 24 Speed Limit Adherence |  |
| --- | --- |
| Description: | Increase speed limit adherence on portions miles of the freeway system in the region where speeding is a problem so that 70% of the vehicles that are traveling above the speed limit will travel at or below the posted speed limit. The regional travel demand model assumes that 85% of the vehicles travel above the speed limit under free flow conditions. By assigning five police officers for every 20 miles of freeway, it is expected that speeding will drop from 85% to 25% of all vehicles. The relationship between speed and NOx running emission factors indicates that emissions increase over 48 mph. By reducing the number of vehicles traveling faster than the 55 mph or 65 mph posted speed limit, emission reductions can be realized. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Initially the enforcement will be performed by officers, photo radar technology will subsequently be implemented and replace the need for officers.

- New laws or regulations may be required for use of photo radar.

Summary of Impacts (2005):

Daily VT Reduction:		N/A

Daily VMT Reduction:		N/A

Daily NOx Reductions:		0.8685 tons/day

Emission Analysis:

This measure consists of increased enforcement of the existing speed limits on freeways and expressways throughout the region. The TERM is designed to increase the 15% adherence rate to 70%. The impact of the measure on the emissions calculation process is to reassign additional speeding VMT in the 60, 65 and 70 mph bins to the 55 mph conditions when the posted speed limit is 55 mph. Similarly 65 and 70 mph bins are assigned to the 60 mph bin when the speed limit is 60 mph and 70 mph bin is assigned to the 65 mph bin when the posted speed limit is 65 mph.   The following table shows the NOx emissions benefit estimation for facilities with the three posted speed limits.

NOx Estimation:

| NOx Benefits Due to Speed Limits |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Roadways Posed At 55 mph |  |  |  |  |  |
|  | Before Enforcement |  |  | After Enforcement (55 mph limit) |  | Net Gain in NOX |
|  | Speed (mph) | VMT with 60 mph | NOx Emissions (Tons/day) | Net VMT | NOx Emissions (Tons/day) | Tons/day |
| 1 | 55 | 3818543 | 5.1034 | 6152453 | 8.2226 | -3.1192 |
| 2 | 60 | 891931 | 1.2584 | 267579 | 0.3775 | 0.8808 |
| 3 | 65 | 1683535 | 2.6333 | 505061 | 0.7900 | 1.8433 |
| 4 | 70 | 758691 | 1.4533 | 227607 | 0.4360 | 1.0173 |
|  |  |  | 10.4484 |  | 9.8261 | 0.6223 |
|  | Roadways Posed At 60 mph |  |  |  |  |  |
|  | Before Enforcement |  |  | After Enforcement (60 mph limit) |  | Net Gain in NOX |
|  | Speed (mph) | VMT with 60 mph | NOx Emissions (Tons/day) | Net VMT | NOx Emissions (Tons/day) | Tons/day |
| 1 | 60 | 169500 | 0.2374 | 1111071 | 1.5562 | -1.3188 |
| 2 | 65 | 1237074 | 1.8739 | 371122 | 0.5622 | 1.3117 |
| 3 | 70 | 108028 | 0.1624 | 32408 | 0.0487 | 0.1137 |
|  |  |  | 2.2738 |  | 2.1671 | 0.1067 |
|  | Roadways Posed At 65 mph |  |  |  |  |  |
|  | Before Enforcement |  |  | After Enforcement (65 mph limit) |  | Net Gain in NOX |
|  | Speed (mph) | VMT with 65 mph | NOx Emissions (Tons/day) | Net VMT | NOx Emissions (Tons/day) | Tons/day |
| 1 | 65 | 1506990 | 2.6002 | 3721429 | 6.4211 | -3.8209 |
| 2 | 70 | 3163484 | 5.6577 | 949045 | 1.6973 | 3.9604 |
|  |  |  | 8.2579 |  | 8.1184 | 0.1395 |
|  |  |  |  |  | Total NOx Benefit = | 0.8685 |

# Cost Analysis:

438 miles (219 miles one way x 2) of freeway in the above Jurisdictions

Approximately 5 officers to patrol 20 miles of freeway 24 hours a day

438 miles/20 miles = 22 segments

22 segments x 5 officers = 110 officers

Total cost per officer:

One time cost 		$33,000 (vehicle, equipment, etc)

Maintenance Cost	$10,000 (maintenance cost)

Annual cost		$52,000 (salary, overhead)

### Total			$95,000

Total cost for 150 officers

One time cost		$33,000 x 110 = $3,630,000

Annual cost		$52,800 x 110 = $5,808,000

Maintenance cost 	$10,000 x 110 = $1,100,000

Education		$100,000

TIP cost: 		$6,908,000 x 3 + $3,630,000 +  $100,000 = $24,454,000

Annual cost  = 	$5,808,000 + $1,100,000 +100,000 + $ 3,630,000/6 yrs = $7,613,000

Cost Effectiveness (2005):

| NOx | = | $7,613,000/ year | = | $28,100 / ton |
| --- | --- | --- | --- | --- |
|  |  | 312 days x 0.8685 tons/day |  |  |

| Table 1: Freeway miles by speed and jurisdiction |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Jurisdiction | Route | Freeway Mile |  |  | Total Miles |
|  |  | 55 mph | 60 mph | 65 mph |  |
| Frederick County | I-270, I-70 | 7.0 | 0.0 | 30.4 | 37.4 |
| Mongomary | I-270, I-495 | 31.6 | 0.0 | 4.7 | 36.3 |
| PG County | I-495, I-95, US-50, US295 | 26.9 | 0.0 | 7.0 | 33.9 |
| Firfax | Dulles Toll Rd., I-66, I-495, I-95 | 49.1 | 9.4 | 0.0 | 58.5 |
| Prince Willium | I-66, I-95 | 0.0 | 12.9 | 12.9 | 25.7 |
| Loudoun | Dulles Rd | 2.3 | 0.0 | 9.4 | 11.7 |
| Stafford | I-95 | 0.0 | 0.0 | 15.2 | 15.2 |
|  | ###### Total | 117.0 | 22.2 | 79.6 | 218.8 |

| ## M-47c Employer Outreach for Private Sector Employers |  |
| --- | --- |
| Description: | The adopted TERM provides NOx mitigation through employers by marketing and implementing employer based TDM programs to the private sector. The expanded TERM will provide additional resources for servicing underserved jurisdictions. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- WMATA staff will service all Metro check accounts and then cost will be $350,000. All other sales and service calls will be handled by local sales representatives and central program staff. Their annual expenses will be $500,000.

- Annual funding will be increased by $850,000 from 2003 to 2005.

- The additional funding will be used to focus on District of Columbia, and Prince George’s County.

- The additional funding will increase NOx emission reductions of existing employer outreach program by 40%.

Summary Impact:

| Daily VT Reduction: | 5,303 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 76,894 | VMT |
| Daily NOx Reductions: | 0.0804 | tons/day |

Emission Analysis:

Increase Commuter Connections FY 2001 work program goals for 2005 (interpolated from 2002 and 2020 goals) by 40%:

2005 M-47c goal of trip reduction = 13,223

New 2005 M-47 c goals (40% increase) = 18,526

VT reduced = 18,526 – 13,223 = 5,303

VMT reduced = 5303 x 14.5 = 76,894

Daily NOx Emission Reductions:

| Cold Start | 5303 | x | 0.912 grs | x | 1 ton | = | 0.0053 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 76,894 | x | 0.980 grs | x | 1 ton | = | 0.0831 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.0884 | tons |
| Total emissions reduction with Tier 2 Controls (0.91) |  |  |  |  |  |  | 0.0804 | tons |

## Cost Effectiveness (2005):

Total annual cost = $500,000 + $ 350,000 = $850,000

TIP Cost 2002-2004 = $2,550,000

Daily cost  =  $850,000/250 days  =  $3,400

| NOx Cost Effectiveness: | $850,000 | = $42,300/ton |
| --- | --- | --- |
|  | 250 x 0.0804 |  |

M-47c  Employer Outreach for Private Sector Employers

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 5303 |  | 5680 |  |  |  |  |  |  |
| VMT | 76894 |  | 77924 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 5680 | x | 0.9905 grs | x | 1 ton | = | 0.00620 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 77,924 | x | 0.6995 grs | x | 1 ton | = | 0.06008 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.06629 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 5680 | x | 2.3454 grs | x | 1 ton | = | 0.01469 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 77,924 | x | 0.2717 grs | x | 1 ton | = | 0.02334 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.03802 | tons |  |
| Cost effectiveness | $850,000 |  |  | = | 51293 | $/ton |  |  |  |
| NOx | 250 | x | 0.06629 |  |  |  |  |  |  |
| Cost effectiveness | $850,000 |  |  | = | 89417 | $/ton |  |  |  |
| VOC | 250 | x | 0.03802 |  |  |  |  |  |  |

| ## M – 92 Telecommuting Program |  |
| --- | --- |
| Description: | This TERM looks into actively encouraging employers (government/private sector) to establish telecommuting programs for their employees, and provide planning assistance and other technical expertise towards successful implementation of telecommuting programs and telework center around the region. The TERM would allocate additional funding to promote telecommuting and area telework centers, including legislative changes and financial incentives. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- The Metropolitan Washington Telework Resource Center (MWTRC) will expand its marketing activity as well as provide consultant support for starting telework programs.

- Currently 15% of the workforce telecommutes on an average of 1.5 days per week.

- This analysis assumes that the region will increase the market share from 15% to 20%.

Surveys have shown that 15% of workforce is telecommuting. Reaching the 20% goal by 2005 is achievable. The region would take credit for all telecommuting in the region.

- Commuter Connector surveys have shown that at a minimum 10% of the regional impact is due to the MWTRC workforce.

- For purpose of cost effectiveness calculation, it will be assumed that only 10% of the emissions benefit is achieved on a per day basis from the MWTRC.

- After achieving the 20% telework participation the program will maintain the 20% of the workforce in the program in future years.

#### Emission reductions for the telecommuting program include vehicle trips related to non-work activities. In few instances travel reductions are higher than work travel they eliminated, and, in other cases the travel reductions are less than the work related travel they eliminate.  Under the most conservative approach, telecommuters will save only one trip per day and VMT savings will be 90% of the work VMT.

- The $300,000 will be spent on expanding the outreach of the existing MWTRC program.

# Analysis

| Analysis Year | 2005 |
| --- | --- |
| Number of Jobs | 3,072,200 |
| Total telecommuters (20%) | 614,440 |
| Frequency Days/Week 1.5/5 | 1.5/5 |
| Telecommuters/day | 184,332 |
| 1996 Telecommuters/day | - 40,263 |
| New Telecommuters/day | 144,069 |
| SOV Telecommuters/day | 69729 |

Work Related Trips

| VT reduced | 139,459 |
| --- | --- |
| VMT reduced | 2,538,150 |
| VOC reduced | 1.282748 |
| NOX reduced | 2.622686 |

Non-Work Related Trips

| VT reduced | 69729 |
| --- | --- |
| VMT reduced | 2284335 |
| VOC reduced | 1.018063 |
| NOX reduced | 2.309385 |

#### Additional NOx Reduction for 2005:

# Emission benefits from current program = 1.5709 (credit taken in tracking sheet line item #75)

# NOx Reduction for 2005 = 2.3093 – 1.5709 = 0.738 tons/day

### Cost Analysis:

Expansion and implementation of the telework program would cost $300,000 annually.

Cost Effectiveness (2005):

| NOx = | $300,000 | = | $16,250 / ton |
| --- | --- | --- | --- |
|  | 250 days x 0.738 tons/day x 0.1 |  |  |

| ## M-93 Improve Pedestrian Facilities Near Rail Stations |  |
| --- | --- |
| Description: | This measure assumes improvements to sidewalks, curb ramps, crosswalks, and lighting in order to improve pedestrian access to 11 MARC stations and 12 Metrorail stations in Montgomery County. The analysis was conducted based on needs shown in a Maryland Department of Transportation/ Mass Transit Administration report published in June, 1997 (Access 2000 – Bicycle and Pedestrian Access to Rail Transit Stations in Maryland). |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Made all improvements to sidewalks, curb ramps, crosswalks, and lighting at and around (.6 mile radius from station) Montgomery County rail stations, as needed according to MDOT/MTA’s Access 2000 study.

- Used 2 % growth rate, compounded annually, to grow 1996 average weekday passenger totals to 2005 ridership numbers at each station.  1996 average weekday passenger totals from MDOT/MTA’s Access 2000 study.

- Used 5% increase in passengers for areas that needed improvements at and around (.6 mile radius) the station, and used 3 % increase in passengers for areas that need improvements just around (.6 mile radius) the station.

- Average trip length = 14.5 miles

- 85 percent of new users drive alone prior to using transit

Summary of Impacts (2005)

Daily VT Affected:		4,318 trips

Daily VMT Affected:		62,611 miles/ day

Daily NOx Reductions:		0.0655 tons/day

Emission Analysis (2005)

Increase in pedestrian due to improvements = 78556 – 76014 = 2542 (refer table 1)

Increase in trips = 2 x 2542 = 5084

Number of SOV trips = 5080 x 0.85 = 4318

VMT: 4318 x 14.5 = 62611

Daily Emissions Reductions:

| Cold Start | 4318 | x | 0.912 grs | x | 1 ton | = | 0.00434 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 62,611 | x | 0.980 grs | x | 1 ton | = | 0.06764 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.07198 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.06550 | tons |

Cost Assumptions:

In order to estimate the project cost, staff estimated the miles of sidewalk, and number of intersections requiring curb cuts, wheel chair ramps, crosswalks and signal components at 22 Metro and MARC stations. The cost of the improved pedestrian facilities is estimated at $11.4 million. A contingency cost of  $2.8 million, (25% of estimated cost) was added to the estimated cost. The total cost of improved pedestrian facilities including contingency cost is estimated at $14.2 million. The cost of the project will be spread over FY 2003 through FY 2005.

Cost Analysis:

Cost of improved pedestrian facilities = $11.4 million

Contingency cost 25% = $2.8 million

Life span: 10 years

Number of days per year = 250

Total TIP Cost = $14.2 million

Cost Effectiveness:

| NOx = | $14.2 million | = $87,500/ton |
| --- | --- | --- |
|  | 250 days x 10 yrs x 0.065 tons/day |  |

Tabel 1: Pedestrian Improvements to Transit

|  |  | 1996 | Needs | Grow to | Increase due to |
| --- | --- | --- | --- | --- | --- |
|  |  | Avg. Weekday | improvs. | 2005 | improvement |
|  | Station | Passengers | at station? | 2% comp. Ann. | 3 % or '5% growth |
| MARC: | Dickerson | 12 | Y | 14.34 | 15.06 |
|  | Barnesville | 61 | Y | 72.90 | 76.55 |
|  | Boyds | 9 | Y | 10.76 | 11.29 |
|  | Germantown | 345 | Y | 412.31 | 432.92 |
|  | Metropolitan Grove | 152 | Y | 181.65 | 190.74 |
|  | Gaithersburg | 279 | Y | 333.43 | 350.10 |
|  | Washington Grove | 25 | Y | 29.88 | 31.37 |
|  | Rockville | 314 | N | 375.26 | 386.52 |
|  | Garrett Park | 32 | Y | 38.24 | 40.16 |
|  | Kensington | 111 | Y | 132.66 | 139.29 |
|  | Silver Spring | 334 | N | 399.16 | 411.14 |
| Metrorail: | Shady Grove | 8465 | N | 10116.46 | 10419.95 |
|  | Rockville | 3255 | N | 3890.03 | 4006.73 |
|  | Twinbrook | 3511 | N | 4195.97 | 4321.85 |
|  | White Flint | 3425 | N | 4093.19 | 4215.99 |
|  | Grosvenor | 3201 | N | 3825.49 | 3940.26 |
|  | Medical Center | 3545 | N | 4236.60 | 4363.70 |
|  | Bethesda | 7106 | N | 8492.33 | 8747.10 |
|  | Friendship Heights | 7790 | N | 9309.77 | 9589.06 |
|  | Glenmont |  | N | 0.00 | 0.00 |
|  | Wheaton | 5125 | N | 6124.85 | 6308.59 |
|  | Forest Glen | 1798 | N | 2148.78 | 2213.24 |
|  | Silver Spring | 9908 | Y | 11840.98 | 12433.03 |
|  | Takoma | 4802 | N | 5738.83 | 5911.00 |
|  | TOTAL | 63605 |  | 76014 | 78556 |

M-93   Improve Pedestrian Facilities Near Rail Stations

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 4318 |  | 4625 |  |  |  |  |  |  |
| VMT | 62611 |  | 63449 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 4625 | x | 0.9905 grs | x | 1 ton | = | 0.00505 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 63,449 | x | 0.6995 grs | x | 1 ton | = | 0.04892 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.05397 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 4625 | x | 2.3454 grs | x | 1 ton | = | 0.01196 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 63,449 | x | 0.2717 grs | x | 1 ton | = | 0.01900 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.03096 | tons |  |
| Cost effectiveness | $14,200,000 |  |  | = | 105236 | $/ton |  |  |  |
| NOx | 250 x 10 | x | 0.05397 |  |  |  |  |  |  |
| Cost effectiveness | $14,200,000 |  |  | = | 183456 | $/ton |  |  |  |
| VOC | 250 x 10 | x | 0.03096 |  |  |  |  |  |  |

| ## M – 103 Taxicab Replacement Program in the District of Columbia |  |
| --- | --- |
| Description: | The District of Columbia adopted a taxicab replacement program in the 1995 CLRP. The program was to be implemented after year 2015 with emission benefits being realized by 2020. This TERM can be accelerated and expanded in order to achieve benefits by 2005. A similar program called the Advanced Technology Vehicle Program has been operational in the Maryland suburbs and has successfully introduced compressed natural gas (CNG) powered taxicabs, airport shuttles, personnel transit buses, and school buses. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- 150 cabs will be replaced (50 per year) with new Compressed Natural Gas (CNG) powered vehicles by year 2005.

- Replacement vehicle will meet ULEV standards.

- Cabs to be replaced are at least seven years old with high mileage.

- Mileage accumulation of each vehicle is greater than 50,000 miles per year, which translates to 162 daily VMT per cab.

- Taxicabs will be operational 312 days per year.

- Replacement vehicles will not be used after 6 years.

- CNG replacement vehicles will meet Low Emission Vehicle (LEV) standards.

- New CNG vehicles will cost $25,000 each.

- Agreement between administering agency and cab operator to achieve stated emission benefits.

- Administration of the cab replacement program will cost $200,000 annually.

- The program will fund two additional fueling facilities and refurbishment of existing CNG stations that need upgrading.

Emission Reductions:

| TERM ID | Replacement Vehicle Type | 2005 Emission Reductions (tons / day) |  |
| --- | --- | --- | --- |
|  |  | VOC | NOx |
| M-103 | CNG | 0.1307 | 0.203 |

Cost Analysis:

# CNG Replacement Vehicles

Total Cost of 150 cabs     	$25,000 x 150  =  $3,750,000

Administrative cost 		$200,000/yr for 3 years

Refueling facility cost 		$1,000,000

Total TIP cost 			$3,750,000 + $1,000,000 + $600,000  = $5,350,000

Annualized Cost = $3,750,000 /6 + $1,000,000/15 + $200,000 = $891,666

Cost Effectiveness (2005):

NOx  =  $891,666 / (312 days/year x 0.203 tons NOx/day) = $14,300/ ton

| ## M – 103a Taxicab Replacement Program in the District of Columbia ## (Conventional vehicle) |  |
| --- | --- |
| Description: | The District of Columbia adopted a taxicab replacement program in the 1995 CLRP. The program was to be implemented after year 2015 with emission benefits being realized by 2020. This TERM can be accelerated and expanded in order to achieve benefits by 2005. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- 150 cabs will be replaced (50 per year) with new “conventional” powered vehicles by year 2005.

- Conventional replacement vehicles will meet Light Duty Gasoline Vehicles (LDGV) standards.

- Cabs to be replaced are at least seven years old with high mileage.

- Mileage accumulation of each vehicle is greater than 50,000 miles per year, which translates to 162 daily VMT per cab.

- Taxicabs will be operational 312 days per year.

- Life span of the new cars- 6 years

- New LDGV vehicles will cost $21,000 each.

- Operation of the cab replacement program will cost $200,000 annually.

Emission Reductions:

| TERM ID | Replacement Vehicle Type | 2005 Emission Reductions (tons / day) |  |
| --- | --- | --- | --- |
|  |  | VOC | NOx |
| M-103a | LDGV | 0.1136 | 0.1751 |

Cost Analysis:

LDGV Replacement Vehicles:

Total Cost of 2005 Reductions:

3 years x (50 x $21,000 years + $200,000 / year) = $3,750,000

Annualized Cost:

150 x $21,000 / 6 years + $200,000 / year = $725,000

2005 Cost Effectiveness:

$725,000 / (312 days/year x 0.175 tons /day) = $13,300 / ton NOx

| ## M-110 10 Transit Stores in Maryland |  |
| --- | --- |
| Description: | This measure would establish 10 Transit Stores in Maryland, similar to those in Arlington County, Virginia and Silver Spring, Maryland. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Implementation time for the first transit store is 1 year.  The planning costs are estimated to be $27,500 and planning is assumed for 2002.

- Each transit store is assumed to be in operation for one year before any impacts occur.  The first transit store is planned for 2002, with first benefits occurring in 2003.

- It is estimated to take 5 years for each transit store to reach full potential.

- Initial set-up costs for each transit store are $6,050 (one-time)

- Annual operating costs for each transit store are $126,500

- Exact locations of the transit stores will be decided by the state agencies and local jurisdictions.

- Ridesharing analysis is based on the Commuter Connections Transportation Demand Management Evaluation Project, January 10, 2000. Transit analysis is based on Arlington county transit store surveys.

- Estimated number of applicants a transit store would serve is 1,200 a month and 14,400 per year.  This is based on average monthly visitors to the transit store in Silver Spring (April 2001)

Summary of Impacts (2005)

| Daily VT Reduction: | 1095 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 31,705 | VMT |
| Daily NOx Reductions: | 0.0322 | tons/day |

ANALYSIS FOR A SINGLE TRANSIT STORE

Calculation of New Transit Riders

The results of the Arlington transit store survey conclude the following characteristics of those applicants entering the store:

| Full-Time Employees: | 70% |
| --- | --- |
| First Time Visitors: | 35% |
| Non-Transit Users: | 5% |
| Potential to become Transit Users: | 79% |

These factors are then multiplied together to factor the total applicants who will be assisted.

Population of store visitors who are full-time employees (FTE)	=14,400 x 0.7 = 10,080

Population of store visitors who are FTE and and first-time visitors (FTV)

= 10,080 x 0.7 x 0.35

=3,528

Population of store visitors who are FTE and FTV, and who are currently not using Transit (CNUT)

= 14,400 x 0.7 x 0.35 x 0.05

= 176

Population of store visitors who are FTE, FTV, and who are likely to use transit

= 42,000 x 0.7 x 0.35 x 0.05 x 0.79

= 139

= 139 x 2 trips = 278 two-way trips

Data from the MWCOG study 1991, Passenger Access to Suburban Metrorail Stations was used to determine the percent of the new transit riders that will be accessing transit by automobile.  In such cases, the VMT from the home site to the transit station will need to be accounted.

| ### Access to Transit |  |  |  |  |
| --- | --- | --- | --- | --- |
| Auto | 74% |  |  |  |
| Bus | 17% |  |  | 24% |
| Walk | 7% |  |  |  |
| Other | 3% |  |  |  |

# VT and VMT Reduction from New Transit Riders

| New Transit Riders | 278 |
| --- | --- |
| Bus & Walk Access | 24% |
| Avg. Vehicle Occupancy | 1.1 |
| Walk Access VT Reduction | 61 |

| 61 (Walk Access VT Red.) | x | 14.5 miles (avg. trip length) x 2 | = | 1,769 VMT Reduced |
| --- | --- | --- | --- | --- |

Although the primary function of the transit stores will be to assist commuters into transit, each store will also be on-line with the Commuter Connections program and be able to assist commuters into pooling arrangement as well.

Calculation of New Poolers

The calculation of new rideshare commuters was based on information derived from the MWCOG study: 1993 Survey and Evaluation of Ride Finders Ridesharing Network.  The number of applicants was based on the total of 5,356 rideshare applications for the Maryland rideshare programs (Montgomery county, Prince George’s county and Frederick county).  The impacts of the 10 transit stores were assumed to be 20% of the total Maryland ridesharing programs. This calculates to approximately 1,071 new applicants over all ten stores or 107 new applicant at each store each year.

All applicants placement rates:

Continued placement rate: 10.6%

Temporary placement rate: 4.9%

One-time placement rate: 27.3%

Calculations for VT and VMT

Continued placement 107 x 10.6% = 11.34

Temporary placement 107 x 4.9% = 5.24

One-time placement 107 x 27.3% = 29.21

Total = 46

Total daily VT reduced = 46 VT

VMT reduction = 46 x 2 x 14.5 = 1,334 VMT

| Ridesharing Attrition |  |  |  |
| --- | --- | --- | --- |
|  | 1st year of operation | 2nd year of operation | 3rd year of operation |
| 1st year applicants | 100.0% | 100.0% | 50.0% |
| 2nd year applicants |  | 100.0% | 100.0% |
| 3rd year applicants |  |  | 100.0% |
| TOTAL | 100.0% | 200.0% | 250.0% |

| Transit Commuter Attrition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st year of operation | 2nd year of operation | 3rd year of operation | 4th year of operation | 5th year of operation |
| 1st year applicants | 100.00% | 50.00% | 25.00% | 12.50% | 6.25% |
| 2nd year applicants |  | 100.00% | 50.00% | 25.00% | 12.50% |
| 3rd year applicants |  |  | 100.00% | 50.00% | 25.00% |
| 4th year applicants |  |  |  | 100.00% | 50.00% |
| 5th year applicants |  |  |  |  | 100.00% |
| TOTAL | 100.00% | 150.00% | 175% | 187.50% | 193.75% |

The following table illustrates the VT and VMT reduction of a single transit store when attrition is accounted.

|  | 1st Year | 2nd Year | 3rd Year | 4th Year | 5th Year |
| --- | --- | --- | --- | --- | --- |
| CAR/VANPOOL |  |  |  |  |  |
| attrition rate | (100%) | (200%) | (250%) | (250%) | (250%) |
| VT Reduction | 46 | 92 | 115 | 115 | 115 |
| VMT Reduction | 1334 | 2668 | 3335 | 3335 | 3335 |
| TRANSIT |  |  |  |  |  |
| attrition rate | (100.00%) | (150.00%) | (175.00%) | (187.50%) | (193.75%) |
| VT Reduction | 61 | 92 | 107 | 114 | 118 |
| VMT Reduction | 1769 | 2654 | 3095 | 3317 | 3427 |
| ###### T O T A L |  |  |  |  |  |
| VT Reduction | 107 | 184 | 222 | 229 | 233 |
| VMT Reduction | 3103 | 5322 | 6430 | 6652 | 6762 |

2005 Cost Analysis for a SINGLE transit store

The following table illustrates the costs associated with this measure which would occur in the FY 2000-2005 TIP.  It is estimated to take approximately five years of operation before the transit store reaches its full potential.

| One-Time Costs (Annualized for cost effectiveness calculations) |  |  |
| --- | --- | --- |
| Planning Costs | $4,580 | One-sixth of the actual planning costs of $27,500 |
| Commuter Connections | $366 | One-sixth of the computers costs ($2,200) |
| Pentium PC/ Printer | $640 | One-sixth of the software costs ($3,850) |
| Total One-Time Costs | $5,586 |  |
| Annual Operating Costs |  |  |
| Salaries/General Overhead | $126,500 | The first 2 stores (Mont. Co.) will assume no cost |
| Monitoring Costs | $27,500 |  |
| Commuter Connections | $3,300 |  |
| Total Annual Operating Costs | $157,300 |  |
| Total Costs | $162,886 |  |

The daily NOx reduction for a SINGLE Transit Store in 2005 is 0.0085 tons per day.  This accounts for  attrition as well as a VT growth factor of 1.0175 and a VMT growth factor of 1.0198 between 2000 and 2020.

Daily Emissions Reductions For 10 Transit Store (2005)

| Cold Start | 1095 | x | 0.912 grs | x | 1 ton | = | 0.0011 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |
| Running | 31,705 | x | 0.980 grs | x | 1 ton | = | 0.0342 |
|  |  |  | 1 mi |  | 907,185 |  |  |
|  |  |  |  |  | Total |  | 0.0354 |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.0322 |

The cost effectiveness is calculated for 2005 to be consistent with other measures being analyzed.

Detailed Cost Analysis for the FY 2000-2005 TIP

|  |  |  | FY 2002 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # One-Time Operating Costs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| First Yr. Planning Costs |  |  |  |  | $27,500 |  |  |  |  |  |  |  |  |  |  |  |
| Software and Installation |  |  |  |  | $2,200 |  | $2,200 |  |  | $2,200 |  | $2,200 |  | $2,200 |  | $2,200 |
| Computer Costs |  |  |  |  | $3,850 |  | $3,850 |  |  | $3,850 |  | $3,850 |  | $3,850 |  | $3,850 |
| Total One-Time Costs |  |  |  |  | $33,550 |  | $6,050 |  |  | $6,050 |  | $6,050 |  | $6,050 |  | $6,050 |
| Annual Operating Costs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Salaries/General Overhead |  |  |  |  | $0 |  | $0 |  |  | $126,500 |  | $253,000 |  | $379,500 |  | $506,000 |
| Monitoring Costs |  |  |  |  | $27,500 |  | $55,000 |  |  | $82,500 |  | $110,000 |  | $137,500 |  | $165,000 |
| Commuter Connections |  |  |  |  | $3,300 |  | $6,600 |  |  | $9,900 |  | $13,200 |  | $16,500 |  | $19,800 |
| Total Operating Costs |  |  |  |  | $30,800 |  | $61,600 |  |  | $218,900 |  | $376,200 |  | $533,500 |  | $690,800 |
| TOTAL COSTS |  |  |  |  | $64,350 |  | $67,650 |  |  | $224,950 |  | $382,250 |  | $539,550 |  | $696,850 |
| Cost Effectiveness for a Single Transit Store (2005) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOx | = | $162,886 |  |  |  |  |  |  |  |  | = |  | $64,960/ton |  |  |  |
|  |  | 295 days |  | x |  | 1 year |  | x | 0.0085 tons |  |  |  |  |  |  |  |

#### TIP Cost: $64,300 + 4 x $30,800 + 3 x $6,050 + 7 x $30,800 + 3 x $6050 = $439,100

#### ANALYSIS FOR 10 TRANSIT STORES

It was estimated that each transit store will need to be in operation for one year to be able to reach the potential VT and VMT reduction, and for approximately 5 years before full potential is reached.

The following table illustrates the anticipated year the transit store will open and the year benefits are expected.

|  | Funding Available By | Date of Operation | First Benefits |
| --- | --- | --- | --- |
| 1st Store | 2001 | 2002 | 2003 |
| 2nd Store | 2002 | 2003 | 2004 |
| 3rd Store | 2002 | 2003 | 2004 |
| 4th Store | 2002 | 2003 | 2004 |
| 5th Store | 2003 | 2004 | 2005 |
| 6th Store | 2003 | 2004 | 2005 |
| 7th Store | 2003 | 2004 | 2005 |
| 8th Store | 2004 | 2005 | 2006 |
| 9th Store | 2004 | 2005 | 2006 |
| 10th Store | 2004 | 2005 | 2006 |

Sources of information:

Arlington County Commuter Assistance Program, second quarterly report FY 1996.

Transit Store-In Store-Research Study, The Marketing Source, May 10, 1995.

Survey and Evaluation of Ride Finders Ridesharing Network, 1993.

The following tables illustrate the VT and VMT reduction for each TMO as well as all 10 Transit Stores.  The VT and VMT shown accounts for ridesharing and pooling attrition, as well as a growth factor

| ##### VMT Summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2002 | 2003 | 2004 | 2005 |  |
|  | New Stores | New Stores | New Stores | New Stores | Total VMT |
|  | 1 | 2, 3, 4 | 5, 6, 7 | 8, 9, 10 |  |
| 2002 | 0 |  |  |  |  |
| 2003 | 3103 | 0 |  |  | 3103 |
| 2004 | 5322 | 9309 | 0 |  | 14631 |
| 2005 | 6430 | 15966 | 9309 | 0 | 31705 |
| 2006 | 6652 | 19290 | 15966 | 9309 | 51217 |
| 2007 | 6762 | 19956 | 19290 | 15966 | 61974 |

| ##### VT Summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2002 | 2003 | 2004 | 2005 | Total VT |
|  | New Stores | New Stores | New Stores | New Stores |  |
|  | 1 | 2, 3, 4 | 5, 6, 7 | 8, 9, 10 |  |
| 2002 | 0 |  |  |  | 0 |
| 2003 | 107 | 0 |  |  | 107 |
| 2004 | 184 | 321 | 0 |  | 505 |
| 2005 | 222 | 552 | 321 | 0 | 1095 |
| 2006 | 229 | 666 | 552 | 321 | 1768 |
| 2007 | 233 | 687 | 666 | 552 | 2138 |

## M-110 10 Transit Stores in Maryland

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| By 2005, 3 transit stores will be in operations. Corresponding VT & VMT reduction in 2005 will be 321 and 9303 respectively. |  |  |  |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 321 |  | 344 |  |  |  |  |  |  |
| VMT | 9303 |  | 9428 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 344 | x | 0.9905 grs | x | 1 ton | = | 0.00038 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 9,428 | x | 0.6995 grs | x | 1 ton | = | 0.00727 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00764 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 344 | x | 2.3454 grs | x | 1 ton | = | 0.00089 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 9,428 | x | 0.2717 grs | x | 1 ton | = | 0.00282 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00371 | tons |  |
| Cost of 3 Transit Stores: $64,300 + 4 * $30,800 + 3 * $6,050 = $205,650 |  |  |  |  |  |  |  |  |  |
| Cost effectiveness | $205,650 |  |  | = | 91190 | $/ton |  |  |  |
| NOx | 295 | x | 0.00764 |  |  |  |  |  |  |
| Cost effectiveness | $205,650 |  |  | = | 187776 | $/ton |  |  |  |
| VOC | 295 | x | 0.00371 |  |  |  |  |  |  |

| ## M-111 Replace Traffic Signals with Lesser Controls |  |
| --- | --- |
| # Description: | For purposes of this analysis Traffic Signals with Lesser Controls TERM will be limited to modern Roundabouts. Fundamentally, roundabouts are a form of intersection control. This measure will reduce intersection delay time, reduce accidents, render the intersection safer for pedestrians as well as non-motorized traffic, and passively enforce the posted speed limit. A further benefit that has been observed as a result of roundabouts is the resurgence of vehicular traffic through the intersection where the roundabout is installed. This traffic was using residential collector roads to avoid the prolonged delay caused by the signalized intersection. |
| # Analysis Tool: | Sketch Planning |

Note: Analysis/Methodology for this TERM is under revision.

| ## M-113 6 Kiosks in Maryland |  |
| --- | --- |
| Description: | This measure would establish 6 Transportation Information Kiosks in Maryland (similar to those being placed in Virginia and the District of Columbia). |
| Analysis Tool: | Sketch Planning |

Assumptions:

It should be noted that the placement of the kiosks is of critical importance in achieving the estimated impacts.

- Implementation time for the first kiosk is one year.  Planning costs are estimated to be $33,000 and planning is assumed for 2001.

- Each kiosk is assumed to be in operation for one year before any impacts occur.  The first kiosk is planned for 2002, with first benefits occurring in 2003.

- Hardware & Software costs for each kiosk $55,000 (one-time).

- Annual operating costs for each kiosk is $8,800.

- A one-time marketing expense of $ 22,000/year for two years is assumed for each kiosk.

- Exact locations of the kiosks will be decided during the planning study.

- Each kiosk was assumed to generate 12 applicants/year.  Then number of applicants is based on information collected from Commuter Connections data for similar kiosks in Virginia.

- Calculations for placements are based on the January 10, 2001 Commuter Connections Transportation Demand Management Evaluation Project.

| Summary of Impacts (2005) |  |  |
| --- | --- | --- |
| Daily VT Reduction: | 13 | trips |
| Daily VMT Reduction: | 163 | miles |
| Daily NOx Reduction: | 0.0002 | tons |
| Daily VOC Reduction | 0.00001 | tons |

Analysis for a Single kiosk

Each kiosk generates an average of 12 applicants per year.

All applicants placement rates:

Continued placement rate: 10.6%

Temporary placement rate: 4.9%

One-time placement rate: 27.3%

Calculations for VT and VMT

Continued placement 12 x 10.6% = 1.27

Temporary placement 12 x 4.9% = 0.58

One-time placement 12 x 27.3% = 3.27

Total = 5.12

Total daily VT reduced = 5 VT

VMT reduction = 5 x 2 x 14.5 = 145 VMT

It is assumed that applicants that are placed in a car or vanpool will remain in the pool for approximately 2.5 years on the average.  The following table illustrates the ridesharing attrition.

It was assumed that new commuters who shift into transit will remain as transit riders for a maximum of five years.  It is estimated that for each of those five years only one-half of the existing transit riders will remain.  This is illustrated in the following table.

| Attrition Rate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st year of operation | 2nd year of operation | 3rd year of operation | 4th year of operation | 5th year of operation |
| 1st year applicants | 100.00% | 50.00% | 25.00% | 12.50% | 6.25% |
| 2nd year applicants |  | 100.00% | 50.00% | 25.00% | 12.50% |
| 3rd year applicants |  |  | 100.00% | 50.00% | 25.00% |
| 4th year applicants |  |  |  | 100.00% | 50.00% |
| 5th year applicants |  |  |  |  | 100.00% |
| TOTAL | 100.00% | 150.00% | 175.00% | 187.50% | 193.75% |

Total VT and VMT reduction from kiosks, including transit and ridesharing attrition for a Single Kiosk

|  | Year 1 | Year 2 | Year 3 | Year 4 | Year 5 |
| --- | --- | --- | --- | --- | --- |
| attrition rate | (100.00%) | (150.00%) | (175.00%) | (187.50%) | (193.75%) |
| Total VT Reduction | 5 | 8 | 9 | 10 | 11 |
| Total VMT Reduction | 145 | 218 | 254 | 272 | 281 |

The following table illustrates the costs associated with this measure which would occur in the FY 2000-2005 TIP.  It is assumed that a kiosk would be in place each year from 2002 to 2006 (and additional year in 2007).

# 2005 Cost Analysis for a SINGLE kiosk

| One-Time Costs (Annualized for cost effectiveness calculations) |  |  |
| --- | --- | --- |
| Planning Costs | $5,500 | One-sixth of the actual planning costs of $33,000 [six kiosks] |
| Computer Costs (Hardware/Software) | $11,000 | One-fifth of the computers costs ($55,000) [depreciation] |
| Marketing Costs | $22,000 | One-half of the marketing costs ($44,000) [two year] |
| Total One-Time Costs | $38,500 |  |
| Annual Operating Costs |  |  |
| Telephone, Electricity, etc. | $8,800 |  |
| Rent (when applicable) | $11,000 |  |
| Labor | $10,000 |  |
| Total Annual Operating Costs | $29,800 |  |
| Total Costs | $68,300 |  |

Daily Emissions Reductions  for a single kiosk (2005)

Daily NOx Emissions Reductions

| Cold Start: | 13 trips | x | 0.912 grams | x | 1 ton | = | 0.000013 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 trip |  | 907,185 grams |  |  |
| Running: | 163 mi | x | 0.980 grams | x | 1 ton | = | 0.000176 tons |
|  |  |  | 1 mile |  | 907,185 grams |  |  |
|  | TOTAL |  |  |  |  |  | 0.000189 tons |
|  | Tier 2 factor (0.91) |  |  |  |  |  | 0.000172 tons |

Daily VOC Emissions Reductions (2005)

| Cold Start/ Hot Soak: | 13 trips | x | 2.287 grams | x | 1 trip |  | 0.000033 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mile |  | 907,185 grams |  |  |
|  |  |  |  | x |  |  |  |
| Running: | 163 mi | x | 0.347 grams |  | 1 ton | = | 0.000062 tons |
|  |  |  | 1 mile |  | 907,185 grams |  |  |
|  | TOTAL |  |  |  |  |  | 0.000095 tons |
|  | Tier 2 factor (0.97) |  |  |  |  |  | 0.000092 tons |

# Detailed Cost Analysis for the FY 2002-2007 TIP

|  | FY 2002 | FY 03 | FY 04 | FY 05 | FY 06 | FY 07 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of Kiosks | 1 | 2 | 3 | 4 | 5 | 6 |
| One-Time Operating Costs |  |  |  |  |  |  |
| First Yr. Planning Costs | $33,000 |  |  |  |  |  |
| Hardware & Software | $55,000 | $55,000 | $55,000 | $55,000 | $55,000 | $55,000 |
| Marketing | $22,000 | $44,000 | $44,000 | $44,000 | $44,000 | $44,000 |
| Total One-Time Costs | $110,000 | $99,000 | $99,000 | $99,000 | $99,000 | $99,000 |
| Annual Operating Costs |  |  |  |  |  |  |
| Electricity/Telephone | $8,800 | $17,600 | $26,400 | $35,200 | $44,000 | $52,800 |
| Labor | $10,000 | $10,000 | $10,000 | $10,000 | $10,000 | $10,000 |
| Rent | $11,000 | $22,000 | $33,000 | $44,000 | $55,000 | $66,000 |
| Total Operating Costs | $29,800 | $49,600 | $69,400 | $89,200 | $109,000 | $128,800 |
| TOTAL COSTS | $128,000 | $148,600 | $168,400 | $188,200 | $208,000 | $227,800 |

TIP Cost: $128,000 + $148,000 + $168,000 = $444,000

| Cost Effectiveness for a Single kiosk (2005) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx | = | $68,300 |  |  |  |  | = | $1,588,372 /ton |
|  |  | 250 days | x | 1 year | x | 0.000172 tons |  |  |

| VOC | = | $68,300 |  |  |  |  | = | $2,969,565/ton |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 250 days | x | 1 year | x | 0.000092 tons |  |  |

Analysis for 6 Kiosks

It was estimated that each kiosk will need to be in operation for one year to be able to reach the potential VT and VMT reduction, and for approximately 5 years before full potential is reached.

The following table illustrates the anticipated year the TMO will open and the year benefits are expected.

|  | Funding Available By | Date of Operation | First Benefits |
| --- | --- | --- | --- |
| 1st kiosk | 2002 | 2003 | 2004 |
| 2nd kiosk | 2003 | 2004 | 2005 |
| 3rd kiosk | 2004 | 2005 | 2006 |
| 4th kiosk | 2005 | 2006 | 2007 |
| 5th kiosk | 2006 | 2007 | 2008 |
| 6th kiosk | 2007 | 2008 | 2009 |

The following table illustrates the VT and VMT reduction for each kiosk, as well as all 6 kiosks.  The VT and VMT shown account for transit and car/vanpool attrition as well as a growth factor from 2000-2020 of 1.0175 for VT and 1.0198 for VMT.

M-113  6 Kiosks in Maryland

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 13 |  | 14 |  |  |  |  |  |  |
| VMT | 163 |  | 165 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 14 | x | 0.9905 grs | x | 1 ton | = | 0.00002 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 165 | x | 0.6995 grs | x | 1 ton | = | 0.00013 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00014 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 14 | x | 2.3454 grs | x | 1 ton | = | 0.00004 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 165 | x | 0.2717 grs | x | 1 ton | = | 0.00005 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00009 | tons |  |
| Cost effectiveness | $68,300 |  |  | = | 1916240 | $/ton |  |  |  |
| NOx | 250 | x | 0.00014 |  |  |  |  |  |  |
| Cost effectiveness | $68,300 |  |  | = | 3196305 | $/ton |  |  |  |
| VOC | 250 | x | 0.00009 |  |  |  |  |  |  |

## M-123 Employer Outreach for Public Sector Agencies

Description:	The region adopted a employer outreach NOx mitigation measure that was based on marketing and implementing employer based TDM programs to the private sector.  This measure will implement a similar program aimed at the public sector employers and employees in the region.

Analysis Tool:	Sketch Planning

Assumptions:

- Total number of public sector employees who are not in alternative commute mode.

Federal:	323,057 – 137,500 (Metrocheck participants) = 			185,557

State:		  61,694 – 100 (Metrocheck and vanpool users) = 		  61,594

Local:		164,177 – 829 (Metrocheck and vanpool participants) = 		163,348

Carpool/Vanpool Participants 						            -137,500

Total:										273,000

- Number of employees in public sector who are not in alternative commute mode -  273,000

- Baseline participation of employers and employees in TDM programs higher than private sector.

- Same level of effort (as expended with the private sector) required to achieve a 5% reduction in vehicle trips with the public sector.

- Four years to achieve the full 5% reduction. By 2005 only 2.5% reduction will be achieved.

- VT and VMT estimates were based on the previous employer outreach program.

- If start of program in summer of 2003 the first year of benefits will be 2004.

- All Metrochecks leads will be serviced by WMATA. Their estimated expenses are $350,000/year. All other sales and service calls will be handled by local sales representatives and central staff.. Their annual expenses will be $450,000.

# Summary of Impacts (2005):

| Daily VT Reduction: | 13650 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 197925 | VMT |
| Daily NOx Reductions: | 0.207 | tons/day |

# Emission Analysis:

Current travel mode based on state of commute survey.

| Mode | Regional | Average | Rideshare Survey |
| --- | --- | --- | --- |
| Drive Alone | 65% | 51% | 37% |
| Transit | 15% | 20% | 24% |
| Carpool | 15% | 25% | 34% |
| Other | 5% | 4% | 5% |

SOV commuters who will switch mode due to the program = 273,000 x 0.05 = 13,650

SOV commuters who will switch mode by 2005due to the program = 13,650/2 = 6825

VT reduced = 6,825 x 2 trips/day = 13,650

VMT reduced = 13,650 x 14.5 = 197,925

Daily Nox Emissions Reductions (2005)

| Cold Start | 13,650 | x | 0.912 grs | x | 1 ton | = | 0.01372 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 197,925 | x | 0.980 grs | x | 1 ton | = | 0.21381 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.22753 | tons |
| Total emission reductions with Tier 2 Controls (0.91) |  |  |  |  |  |  | 0.20706 | tons |

# Cost Assumptions:

Annual recurring administrative cost	$450,000 + $350,000 = $800,000

Planning cost				$50,000	 one-time cost

Assumed days per year			250 days

Annual Cost  				$800,000 + $50,000/3 = $816,666

Total 2002-04 TIP Cost			$816,666 x 3  = $2,430,000

Cost Effectiveness (2005):

| NOx | = | $816,666 |  |  | = | $15,800 | /ton |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 250 days | x | 0.207 tons |  |  |  |

M-123 Employer Outreach for Public Sector Agencies

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 13650 |  | 14622 |  |  |  |  |  |  |
| VMT | 197925 |  | 200575 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 14622 | x | 0.9905 grs | x | 1 ton | = | 0.01596 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 200,575 | x | 0.6995 grs | x | 1 ton | = | 0.15466 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.17062 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 14622 | x | 2.3454 grs | x | 1 ton | = | 0.03780 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 200,575 | x | 0.2717 grs | x | 1 ton | = | 0.06007 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.09787 | tons |  |
| Cost effectiveness | $816,666 |  |  | = | 19146 | $/ton |  |  |  |
| NOx | 250 | x | 0.17062 |  |  |  |  |  |  |
| Cost effectiveness | $816,666 |  |  | = | 33376 | $/ton |  |  |  |
| VOC | 250 | x | 0.09787 |  |  |  |  |  |  |

| ## M-132 MD/DC Vanpool Incentive Program |  |
| --- | --- |
| Description: | This measure is a package of programs and incentives designed to increase the number of vanpools in the region. The program would work as an extension of the Virginia Vanpool Incentive Program. |
| Analysis Tool: | Sketch Planning |

Assumptions:

This program is similar to the program, which is about to be operational in Virginia.  Vanpools will be provided a monthly lump sum subsidy for reporting information regarding service provided (number of passenger, miles of travel).  Existing vanpools and older vehicles will be provided a reduced subsidy in exchange for reporting data.

Summary of Impacts (2005)

Daily VT reduction:		1,785 VT

Daily VMT reduction:		111,085 miles

Daily NOx reduction		0.1156 tons

Daily VOC reduction		0.04558 tons

Emmission Impacts (2005)

From M-77b Incentive Vanpool in Virginia

Regional VT savings – 7,143 trips

It is assumed that 56% of the vanpools in the region originate in Virginia (1998, COG Beltway Cordon Count).

| New Vans – Region | 371 |  | New Vans MD/DC | 163 |
| --- | --- | --- | --- | --- |
| Used Vans – Region | 978 |  | Used Vans MD/DC | 430 |
| Total Vans – Region | 1,349 |  | Total Vans MD/DC | 593 |

Funding available by FY 	2003

Vanpool by summer 		2004

First benefits  			2005

Average Trip Distance = 31.1 miles (from: Commuter Connections Transportation Demand Management Evaluation Project. LDA Consulting. January 10, 2000)

VMT = 7,143  x 31.1 x  2 = 444,341 miles (2008)

VT (2005) = 7,143/4  =   1,785 VT

VMT (2005) =  444,341/4 = 111,085 miles

Daily NOx emission reductions:

| Cold Start: | 1,785 trips | x | 0.912grams | x | 1 ton | = | 0.00179 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 trip |  | 907,185 grams |  |  |
| Running: | 111,085 mi | x | 0.980grams | x | 1 ton | = | 0.12 tons |
|  |  |  | 1 mile |  | 907,185 grams |  |  |
|  | TOTAL |  |  |  |  |  | 0.1217 tons |
|  | Total emissions reductions with Tier 2 Control (0.91) |  |  |  |  |  | 0.1156 tons |

Daily VOC Reductions (2005)

| Cold Start + Hot Soak | 1,785 trips | x | 2.287 grams | x | 1 ton | = | 0.0045 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 trip |  | 907,185 grams |  |  |
| Running: | 111,085 mi | x | 0.347 grams | x | 1 ton | = | 0.04249tons |
|  |  |  | 1 mile |  | 907,185 grams |  |  |
|  | TOTAL |  |  |  |  |  | 0.04699 tons |
|  | Total emissions reductions with Tier 2 Control (0.97) |  |  |  |  |  | 0.04558 tons |

# Cost Analysis:

New Van subsidy (based on VA model)

| $1563 (subsidy) x 1,000 | = | $ 30.05 day/van |
| --- | --- | --- |
| 250 days x 208 (Vans in VA) |  |  |

Cost of subsidy for newer Vans (MD & DC) = $30.05 x 163 vans x 250 days = $1,224,537

Old van subsidy (based on VA model)

| $360 (subsidy) x 1,000 | = | $ 10.51 day/van |
| --- | --- | --- |
| 250 days x 137 (Vans in VA) |  |  |

25 % of MD/DC older vans are 5 years old or newer and are eligible for subsidy:

Cost of subsidy for older vans in MD/DC

430 Vans x 0.25 = 107 Vans

107 Vans x $ 10.51 x 250 days  = $281,142 (every year)

Project Cost:

Administration:					$105,000

Marketing:					$120,000

Insurance:		($400  x  270 Vans)	$108,000

Postage:					$3,000

New Vans		(163 x $30.05 x 250)	$1,224,600

Older Vans					$281,200

Total :						$1,841,800

Cost Effectiveness:

| NOx | = | $1,841,800 /yr | = | $63,730 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 0.1156 tons |  |  |

| VOC | = | $1,841,800 /yr | = | $161,630 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 0.04558 tons |  |  |

M-132 MD/DC Vanpool Incentive Program

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 1785 |  | 1912 |  |  |  |  |  |  |
| VMT | 111085 |  | 112572 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 1912 | x | 0.9905 grs | x | 1 ton | = | 0.00209 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 112,572 | x | 0.6995 grs | x | 1 ton | = | 0.08680 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.08889 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 1912 | x | 2.3454 grs | x | 1 ton | = | 0.00494 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 112,572 | x | 0.2717 grs | x | 1 ton | = | 0.03372 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.03866 | tons |  |
| Cost effectiveness | $1,841,800 |  |  | = | 82881 | $/ton |  |  |  |
| NOx | 250 | x | 0.08889 |  |  |  |  |  |  |
| Cost effectiveness | $1,841,800 |  |  | = | 190571 | $/ton |  |  |  |
| VOC | 250 | x | 0.03866 |  |  |  |  |  |  |

| ## M-133 Metrorail Feeder Bus Service |  |
| --- | --- |
| Description: | Improve Metrorail feeder bus service at two underutilized park and ride lots and implement a fare buydown program. |
| Analysis Tool: | Sketch Planning |

This measure is anticipated to attract 175 new transit riders daily, thereby reducing vehicle trips.  Total annual cost of this measure is $445,000.00

Assumptions:

New Carrolton Station

- Bowie Park and Ride lot

- 650 spaces, approximately 300 unutilized on a daily basis

- Improve frequency and span of service to the lot (Metrobus routes affected B21, B22, C28).  Headways will be reduced from 15 minutes to 10 minutes.

- Combine timetables (service to lot currently listed on three timetables) .

- Implement AM fare buydown to $0.50 (offered at lot only, not entire route).

- Investigate use of shoulders of John Hanson Highway

- Improve signage to lot on Northview Drive.

- Implement targeted marketing effort.

- Purchase one bus at $100,000/5 year useful life.

- Total Annual Operating Cost:  $185,000

- New ridership:  100 new riders.

Summary of Impacts (2005)

Daily VT reduction:		--

Daily VMT reduction:		6,050 VT

Daily NOx reduction		0.00595 tons

Daily VOC reduction		0.00224 tons

Emmission Impacts (2005)

New Carrolton Station

Trip length: 19 mi o/w * 2 = 38 mi round trip

38 * 100 new riders = 3,800 VMT

No VT savings

Daily NOx emission reductions:

| Running: | 3,800 | x | 0.980 grs | x | 1 ton | = | 0.0041 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.00374 tons |

Daily VOC emission reductions

| Running | 3,800 | x | 0.347 grs | x | 1 ton | = | 0.00145 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |
|  |  |  |  | Tier 2 Controls (0.97) |  |  | 0.00141 tons |

Glenmont Station

- Norbeck Road Park-and-ride lot

- 250 spaces, approximately 225 unutilized on a daily basis.

75 new riders

Trip length:  15 mi o/w * 2 = 30 mi round trip

30 * 75 = 2,250 VMT

No VT savings

Daily NOx emissions reductions

| Running | 2,250 | x | 0.980 grs | x | 1 ton | = | 0.00243 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 |  |  |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.00221 tons |

Daily VOC emissions reductions

| Running | 2,250 | x | 0.347 grs | x | 1 ton | = | 0.0086 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 |  |  |
|  |  |  |  | Tier 2 Controls (0.97) |  |  | 0.00083 tons |

Total NOx:

| New Carrolton | 0.00374 tons |
| --- | --- |
| Glenmont | 0.00221 tons |
| Total | 0.00595 tons |

Total VOC:

| New Carrolton | 0.00141 tons |
| --- | --- |
| Glenmont | 0.00083 tons |
| Total | 0.00224 tons |

Benefits:

| Year | 2002 | 2003 | 2004 | 2005 |
| --- | --- | --- | --- | --- |
| New Carrolton |  | 50 riders | 100 riders | 100 riders |
| Glenmont |  | 37 riders | 75 riders | 75 riders |

Costs:

| ### Year | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 |
| --- | --- | --- | --- | --- | --- | --- |
| Capitol Cost | $300,000 |  |  |  |  |  |
| Operational Cost |  | $385,000 | $385,000 | $385,000 | $385,000 | $385,000 |

Annual Cost:

| $300,000 | = | $ 60,000/yr |
| --- | --- | --- |
| 5 yrs. |  |  |
|  | + | $ 385,000/yr |
|  |  | $ 445,000/yr |

TIP Cost:  $300,000 + $385,000 x 2 = $1,070,000

Cost Effectiveness:

| NOx | = | $ 445,000 /yr | = | $ 299,150 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 yr x 0.00595 |  |  |

| VOC | = | $ 445,000 / yr | = | $ 794,650 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 yr x 0.00224 |  |  |

M-133 Metrorail Feeder Bus Service

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 0 |  | 0 |  |  |  |  |  |  |
| VMT | 1650 |  | 1672 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 0 | x | 0.9905 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 1,672 | x | 0.6995 grs | x | 1 ton | = | 0.00129 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00129 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 0 | x | 2.3454 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 1,672 | x | 0.2717 grs | x | 1 ton | = | 0.00050 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00050 | tons |  |
| Cost effectiveness | $445,000 |  |  | = | 1380600 | $/ton |  |  |  |
| NOx | 250 | x | 0.00129 |  |  |  |  |  |  |
| Cost effectiveness | $445,000 |  |  | = | 3554397 | $/ton |  |  |  |
| VOC | 250 | x | 0.00050 |  |  |  |  |  |  |

| ## M –134 Implement 10 Neighborhood Circulator Bus Service to Metrorail |  |
| --- | --- |
| Description: | The circulator bus service would operate over an expanded period from 5:30 am to 10:00 am and from 3:00 pm to 8:00 pm on weekdays. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Two buses per neighborhood will be required at a cost of $100,000 per bus, with a useful life of 5 years.

- Anticipated ridership is 150 riders per day per circulator, for a total of 1,500 additional transit riders per day.

- The stations where circulator service could be implemented include:

- Cheverly station

- Deanwood station

- Minnesota Ave. station

- Vienna/Fairfax – GMU station

- Dunn Loring – Merrifield station

- Greenbel station

- Van Dorn Street station

- Addison Road station

- Glenmont station

- Rhode Island Ave. station

- New Carrolton

Summary of Impacts (2005)

Daily VT Reduction		3,000 VT

Daily VMT Reduction:		42,000 VMT

Daily NOx Reduction:		0.04403 tons

Daily VOC Reduction		0.02292 tons

Emission Impacts (2005):

Anticipated ridership = 150 riders/day/circulator

150 * 10 = 1,500 additional riders

1 rider = 2 trips

1,500 * 2 = 3,000 VT  @ 14 mi round trip

3,000 * 14 = 42,000 VMT

Daily NOx emissions reductions:

| Cold Start | 3,000 | x | 0.912 grs | x | 1 ton | = | 0.00302 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |
| Running | 42,000 | x | 0.980 grs | x | 1 ton | = | 0.04537 tons |
|  |  |  | 1 mi |  | 907,185 |  |  |
|  |  |  |  |  | Total |  | 0.04839 tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.04403 tons |

Daily VOC emissions reductions:

| Cold Start | 3,000 | x | 2.287 grs | x | 1 ton | = | 0.00756 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
| + Hot Soak |  |  | 1 trip |  | 907,185 grs |  |  |
| Running | 42,000 | x | 0.347 grs | x | 1 ton | = | 0.01607 tons |
|  |  |  | 1 mi |  | 907,185 grs |  |  |
|  |  |  |  |  | Total |  | 0.02363 tons |
|  |  |  |  | Tier 2 Controls (0.97) |  |  | 0.02292 tons |

Cost Analysis:

Annual Operating Costs		$3,000,000/ yr

Capital Costs			$100,000/ bus * 10 = $ 1,000,000

| ### Year | 2003 | 2004 | 2005 | 2006 |
| --- | --- | --- | --- | --- |
| Capital Costs | 1,000,000 |  |  |  |
| Annual Op. Costs |  | 3,000,000 | 3,000,000 | 3,000,000 |

| Capital Costs | $1,000,000 | = | $200,000 / yr |
| --- | --- | --- | --- |
|  | 5 yrs |  |  |
| Operational Costs |  | + | $3,000,000 / yr |
|  |  |  | $3,200,000/ yr |

TIP Cost (2003-2005): $1,000,000 + 2 x $3,000,000 = $7,000,000

Cost Effectiveness (2005):

| NOx | = | $3,200,000 | = | $ 264,500 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 x0.04839 |  |  |

| VOC | = | $3,200,000 | = | $ 541,600 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 0.x 0.0236 |  |  |

M –134  Implement 10 Neighborhood Circulator Bus Service to Metrorail

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 3000 |  | 3214 |  |  |  |  |  |  |
| VMT | 42000 |  | 42562 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 3214 | x | 0.9905 grs | x | 1 ton | = | 0.00351 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 42,562 | x | 0.6995 grs | x | 1 ton | = | 0.03282 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.03633 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 3214 | x | 2.3454 grs | x | 1 ton | = | 0.00831 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 42,562 | x | 0.2717 grs | x | 1 ton | = | 0.01275 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.02106 | tons |  |
| Cost effectiveness | $3,200,000 |  |  | = | 352354 | $/ton |  |  |  |
| NOx | 250 | x | 0.03633 |  |  |  |  |  |  |
| Cost effectiveness | $3,200,000 |  |  | = | 607918 | $/ton |  |  |  |
| VOC | 250 | x | 0.02106 |  |  |  |  |  |  |

| ## M –135 Construction of 5950 Additional Parking at WMATA Metrorail Stations |  |
| --- | --- |
| Description: | WMATA will add a total of 5950 spaces at different Metrorail stations through out the region. |
| Analysis Tool: | Sketch Planning |

Emission Impacts for 5950 Parking Spaces (2005):

Daily VT Reduction:		-

Daily VMT Reduction:		111,080 VMT

Daily NOx Reductions:		0.109 tons

Emission Analysis:

5950 additional spaces

Trip length: 14 mi o/w * 2 = 28 mi round trip

2/3 new trips = 3,967 trips

3967 * 28 = 111,080 VMT

Daily NOx emission reductions (2005):

| Running | 111,080 | x | 0.980 grs | x | 1 ton | = | 0.1200 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.1092 | tons |

Cost Analysis:

| Metrorail Parking Needs for Short-Range TERMS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stations | Line | Gross Spaces | Net Spaces | Preliminary Cost | Comments |
| Branch AVE | Green -F | 1800 | 1500 | $27,625,650 | Construct and |
| Greenbelt | Green - E | 1400 | 1000 | $21,489,650 | replace existing |
| Dunn Loing | Orange - K | 1000 | 700 | $15,353,650 | surface with garage |
| Landover | Orange - D | 1250 | 1000 | $19,188,650 | spaces |
| New Carrolton | Orange - D | 1000 | 750 | $15,353,650 |  |
| Glenmont | Red - B | 1000 | 1000 | $15,353,650 |  |
| Total |  | 7450 | 5950 | $114,364,900 |  |

Annualized Cost:   $114,364,900/30 = $3,812,170 /year

Cost Effectiveness (2005):

| NOx | = | $3,812,170 /year | = | $139,900/ ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 yr x 0.109 |  |  |

M –135  Construction of 5950 Additional Parking at WMATA Metrorail Stations

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 0 |  | 0 |  |  |  |  |  |  |
| VMT | 111080 |  | 112567 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 0 | x | 0.9905 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 112,567 | x | 0.6995 grs | x | 1 ton | = | 0.08680 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.08680 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 0 | x | 2.3454 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 112,567 | x | 0.2717 grs | x | 1 ton | = | 0.03371 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.03371 | tons |  |
| Cost effectiveness | $3,812,170 |  |  | = | 175682 | $/ton |  |  |  |
| NOx | 250 | x | 0.08680 |  |  |  |  |  |  |
| Cost effectiveness | $3,812,170 |  |  | = | 452300 | $/ton |  |  |  |
| VOC | 250 | x | 0.03371 |  |  |  |  |  |  |

| M –142 100 CNG Buses in Place of Diesel Buses (Next Bus Purchase) |  |
| --- | --- |
| Description: | This measure will substitute the purchase of 100 diesel fueled buses with 100 Compressed Natural Gas (CNG) buses. CNG provides further reduction in Particulate Matter, Nitrogen Oxide and Volatile Organic Compound than a new diesel bus. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- 100 heavy duty CNG buses compared to 100 new diesel buses

- 40,000 miles per year.

- 312 days/year operation.

- Baseline emissions calculated for a diesel bus running on Ultra Low Sulfur Diesel (ULSD) fuel.

- Use Mobile 6 emission factors for diesel engines and bhp-hr/mi conversion factor. Particulate Matter calculated using factors obtained from EPA - Update of Heavy-duty Emissions Level (model years 1988-2004+) for Use in Mobile 6.

- CNG emission factors based on EPA certified factors provided by Natural Gas Vehicle Coalition (NGVC).

- WMATA estimates incremental operating cost increase of $12,000 per bus per year.

Summary of Impacts (2005):

Daily VT Reduction:		-

Daily VMT Reduction:		-

Daily NOx Reductions:		0.1358 tons/day

Daily VOC Reductions:		0.0033 tons/day

Emission Analysis (2005)

# Daily NOx Emissions

Diesel

| 3.90 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 100 buses | = | 80.46 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grs/ton |  |  |  |  |  |  |

CNG

| 1.84 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 100 buses | = | 28.88 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grms/ton |  |  |  |  |  |  |

| Diesel | - | CNG | = | ### Total |
| --- | --- | --- | --- | --- |
| 80.46 tons/yr |  | 37.96 tons/yr |  | 42.50 tons/yr |

| Daily Reduction | NOx = | 42.50 tons/year | = | 0.1358 tons/day |
| --- | --- | --- | --- | --- |
|  |  | 313 days/yr |  |  |

Daily VOC Emissions (NMHC – Non Methane Hydro Carbons)

Diesel

| 0.08 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 100 buses | = | 1.6505 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grms/ton |  |  |  |  |  |  |

CNG

| 0.03 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 100 buses | = | 0.6189 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grms/ton |  |  |  |  |  |  |

| Diesel | - | CNG | = | ### Total |
| --- | --- | --- | --- | --- |
| 1.650 tons/yr |  | 0.6189 tons/yr |  | 1.0315 tons/yr |

| 1.0315 tons/year | = | 0.0033 tons/day |
| --- | --- | --- |
| 313 days/yr |  |  |

Cost Analysis:

Incremental cost for a CNG bus as compared to a standard diesel bus: 1 bus:	$50,000

Incremental cost for 100 CNG buses:  						$5,000,000

Incremental operating cost:  $12,000 x 100					$1,200,000

Upgrading an existing refueling facility/garage:					$400,000

Annual cost = $5,000,000/15 + $1,200,000 + $400,000/30	 		$1,546,666

Cost Effectiveness (2005):

| NOx | = | $1,546,666 | = | $36,500/ ton |
| --- | --- | --- | --- | --- |
|  |  | 312 days x 0.1358 tons/day |  |  |

| ## M –142a Replace 100 Old Diesel Buses with 100 CNG Buses Before Normal ## Replacement Schedule |  |
| --- | --- |
| Description: | This measure will advance the purchase of 100 old diesel fueled buses (oldest in fleet) with 100 Compressed Natural Gas CNG buses from 2006 to 2005. CNG provides reduction in particulate matter, volatile organic compounds and nitrogen oxides. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- WMATA has funding for 100 diesel buses in FY 2006. Since the purchase is being advanced, there will be financing cost (20%) and incremental cost for CNG buses in lieu of diesel buses.

- 100 heavy duty CNG buses compared to 100 old diesel (10 years or older) buses.

- 40,000 miles per year

- 312 days/year operation

- Baseline emissions calculated for a diesel bus running on Ultra Low Sulfur Diesel fuel.

- Use Mobile 6 emission factors for diesel engines and bhp-hr/mi conversion factor. Particulate Matter calculated using factors obtained from EPA - Update of Heavy-duty Emissions Level (model years 1988-2004+) for Use in Mobile 6.

Summary of Impacts (2005):

Daily VT Reduction:		-

Daily VMT Reduction:		-

Daily NOx Reductions:		0.136  tons/day

Emission Analysis (2005):

Daily NOx Emissions

Diesel

| 3.90 g/bph-hr x 4.679 bph-hr/mi | x | 40,000 mi | x | 100 buses | = | 80.46 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grs/ton |  |  |  |  |  |  |

CNG

| 1.84 g/bph-hr x 4.679 bph-hr/mi | x | 40,000 mi | x | 100 buses | = | 37.96 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grs/ton |  |  |  |  |  |  |

| Diesel 80.46 tons/yr | - | CNG 37.96 tons/yr | = | ### Total 42.50 tons/yr |
| --- | --- | --- | --- | --- |

| Daily Reduction | NOx = | 42.50 tons/year | = | 0.136 tons/day |
| --- | --- | --- | --- | --- |
|  |  | 312 days/yr |  |  |

Cost Analysis:

Incremental cost for a new CNG bus:   $50,000

Incremental cost for 100 CNG buses:   $5,000,000

Financing cost:                                      $6,000,000

($300,000/bus x 100 buses x 20%)

Cost for upgrading refueling facility:   $400,000

Lifespan of a bus:  15 years

Operating cost ($12,000 x 100):	          $1,200,000

Lifespan of fueling facility:  30 years

| Annualized Capital Cost | $11,000,000 | + | $400,000 | + | $1,200,000 | = | $1,946,666/ year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 15 years |  | 30 years |  |  |  |  |

Cost Effectiveness (2005):

| NOx | = | $1,946,666/ year | = | $45,900/ ton |
| --- | --- | --- | --- | --- |
|  |  | 312 x 0.136 tons/day |  |  |

| ## M-142b Opt for CNG for Next Replacement Bus Purchase |  |
| --- | --- |
| Description: | Purchase of CNG buses instead of the 316 clean diesel buses that are programmed and budgeted for Fiscal Years 2003 through 2005. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- 316 diesel funded in WMATA IRP for Fiscal Years 2003 through 2005

- $15 million for construction and garage retrofits

- 316 CNG buses compared to 316 new diesel buses

- 40,000 miles per year

- 312 days/year operation

- Baseline emissions calculated using standard diesel running on Ultra Low Sulfur Diesel Fuel

- Use Mobile 6 emission factors for diesel engines and bhp-hr/mi conversion factor. Update of Heavy-duty Emissions Level (model years 1988-2004+) for Use in Mobile 6.

- Estimated fuel and maintenance cost increase of  $3.8 million per year.

Summary of Impacts (2005)

Daily VT Reduction:		-

Daily VMT Reduction:		-

Daily NOx Reductions:		- 0.43 tons/day

Daily VOC Reductions:		-

Emission Analysis (2005)

Daily NOx Emissions

Diesel

| 3.90 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 316 buses | = | 254.25 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grs/ton |  |  |  |  |  |  |

CNG

| 1.84 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 316 buses | = | 119.96 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grms/ton |  |  |  |  |  |  |

| Diesel | - | CNG | = | ### Total |
| --- | --- | --- | --- | --- |
| 254.25 tons/yr |  | 119.96 tons/yr |  | 134.29 tons/yr |

| Daily Reduction | NOx = | 134.29 tons/year | = | 0.43 tons/day |
| --- | --- | --- | --- | --- |
|  |  | 312 days/yr |  |  |

Cost Analysis

Incremental cost of CNG Diesel bus    =   $50,000

Capital cost for 316 buses = $15.8 mill

Life of buses = 15 years

Retrofit existing fueling facilities =  $400,000

Capital cost for fuelling facility (retrofit + new facilities) = $15.4 mill

Life of fuelling facility  = 30 years

Annual operating cost: 316 x $12,000  =   $3.8 million

Total TIP cost (2002-2005):  $15.8 mill  +  $15.4 mill  +  $3.8  =  $35 mill

Annualized cost = $15.8 mill / 15 + $ 15.4 mill/30 +  $3.8 mill = $5.367 million

Cost Effectiveness (2005)

| NOx | = | $5.367 mill/ year | = | $ 40,000 / ton |
| --- | --- | --- | --- | --- |
|  |  | 312 x 0.43 tons/day |  |  |

| ## M-142c Replace 952 Bus Fleet with CNG |  |
| --- | --- |
| Description: | Under this program WMATA would replace 952 of its ULSD fueled diesel buses with CNG buses. Cost includes CNG fueling facilities for five Metro bus garages and the increase operating and maintenance cost of CNG buses. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- IRP funding is available for years 2003, 2004, 2005. Advance the purchase of 100 buses programmed in 2006 to 2005. Therefore funding is available for 416 diesel buses. The remaining 536 will require full funding and incremental funding of $50,000/bus will be needed to purchase 416 CNG buses. (The advancing will require financing of 100 buses originally programmed for 2006).

- 952  heavy duty CNG buses compared to 952  new diesel buses.

- 4 of WMATA’s bus garages cannot accommodate CNG facilities.

- 40,000 miles per year.

- 312 days/year operation

- Baseline emissions calculated using new diesel running on ultra low sulfur diesel fuel.

- Use Mobile 6 emission factors for diesel engines and bhp-hr/mi conversion factor. Update of Heavy-duty Emissions Level (model years 1988-2004+) for Use in Mobile 6.

Summary of Impacts (2005):

Daily VT Reduction:		-

Daily VMT Reduction:		-

Daily NOx Reductions:		1.98 tons/day

Daily VOC Reductions:		-

Emission Analysis (2005):

a) Buses with average age of 2.5 years =  360

Daily NOx Emissions

Diesel

| 3.90 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 360 buses | = | 289.66 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grs/ton |  |  |  |  |  |  |

CNG

| 1.84 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 546 buses | = | 136.66 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grms/ton |  |  |  |  |  |  |

| Diesel | - | CNG | = | ### Total |
| --- | --- | --- | --- | --- |
| 289.66 tons/yr |  | 136.66 tons/yr |  | 153 tons/yr |

| Daily Reduction | NOx = | 153 tons/year | = | 0.49 tons/day |
| --- | --- | --- | --- | --- |
|  |  | 312 days/yr |  |  |

b) Buses with average age of 7.5 years = 215

Diesel

| 4.56 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 215 buses | = | 202.26 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grs/ton |  |  |  |  |  |  |

CNG

| 1.84 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 215 buses | = | 81.61 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grms/ton |  |  |  |  |  |  |

| Diesel | - | CNG | = | ### Total |
| --- | --- | --- | --- | --- |
| 202.26tons/yr |  | 81.61 tons/yr |  | 120.65 tons/yr |

| Daily Reduction | NOx = | 120.65 tons/year | = | 0.386 tons/day |
| --- | --- | --- | --- | --- |
|  |  | 312 days/yr |  |  |

c) Buses with average age over 10 years =  377

Diesel

| 6.28 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 377 buses | = | 488.45 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grs/ton |  |  |  |  |  |  |

CNG

| 1.84 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 377 buses | = | 143.11 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 grms/ton |  |  |  |  |  |  |

| Diesel | - | CNG | = | ### Total |
| --- | --- | --- | --- | --- |
| 488.45tons/yr |  | 143.11 tons/yr |  | 345.33 tons/yr |

| Daily Reduction | NOx = | 345.33 tons/year | = | 1.106 tons/day |
| --- | --- | --- | --- | --- |
|  |  | 312 days/yr |  |  |

Total daily NOX reduction = 0.49+ 0.386+1.106= 1.982 tons/day

Cost Analysis:

Capital cost

416 buses need incremental funding:  416 x $50,000  = $20,800,000

Financing cost for 100 buses:   100  x  $300,000  x  20%  =  $6,000,000

536 buses need full funding:  $ 350,000  x 536 =  $187,600,000

Capital cost for buses:  $214,400,000

Capital cost for 5 fuelling facilities and garage retrofit: 5 x $15 mill = $75 million

Upgrade one facility =$400,000

Total fuel facility and garage retrofit cost:  $75,400,000

Operating cost of 952 buses: $12,000/bus/year  x  952 buses  =   $11,424,000

Life of buses: 15 years

Life of fuelling facilities: 30 years

Total TIP cost (2002-2005):  $214.4 mill  +  $75.4 mill  +  $11.424 mill  =  $301.2 mill

Annualized cost = $ 214.4 mill / 15 + $75.4 mill/30 + $11.424 mill = $28.23 million

Cost Effectiveness (2005):

| NOx | = | $28.23 million/ year | = | $45,650/ ton |
| --- | --- | --- | --- | --- |
|  |  | 312 x 1.982 tons/day |  |  |

| ## M-142d Replace all Buses over 12 Years Old with CNG Buses |  |
| --- | --- |
| Description: | Replacement of the 716 oldest diesel buses in the WMATA fleet that emit the most NOx and other pollutants with CNG buses. Cost includes two additional CNG fueling/maintenance facilities. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Purchase of  716 buses

- 316 buses in IRP from 2003 to 2005. Advance the purchase of 100 buses funded in 2006 to 2005. Fund purchase of 300 additional buses by 2005.

- 2 refueling and maintenance facilities at $15 million each

- 40,000 miles per year

- 312 days/year operation

- Baseline emissions calculated using Ultra Low Sulfur Diesel (ULSD) fuel buses.

- Use Mobile 6 emission factors for diesel engines 12 years and older and bhp-hr/mi conversion factor. Update of Heavy-duty Emissions Level (model years 1988-2004+) for Use in Mobile 6.

- Diesel buses with ULSD fuel does not reduce NOx

Summary of Impacts (2005)

Daily VT Reduction:		-

Daily VMT Reduction:		-

Daily NOx Reductions:		2.102  tons/day

Daily VOC Reductions:		-

Emission Analysis (2005)

Daily NOx Emissions

Buses with average age over 12 years =  716

Diesel

| 6.28 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 716 buses | = | 927.66 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 g/ton |  |  |  |  |  |  |

CNG

| 1.84 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 716 buses | = | 271.79 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 g/ton |  |  |  |  |  |  |

| Diesel | - | CNG | = | ### Total |
| --- | --- | --- | --- | --- |
| 927.66 tons/yr |  | 271.79 tons/yr |  | 655.86 tons/yr |

| Daily Reduction | NOx = | 655.86 tons/year | = | 2.102 tons/day |
| --- | --- | --- | --- | --- |
|  |  | 312 days/yr |  |  |

Cost Analysis:

Cost of CNG Diesel bus    	  =   $350,000 per bus

Capital cost for 300 buses 	  =  300 x $350,000 = $105,000,000

Incremental cost for 416 buses  	  =  416 * $50,000  =  $20,800,000

Financing cost for 100 buses 	  =  100 x $300,000 x 20% = $6,000,000

Total Capital cost for 716 buses   =  $131,800,000

Life of buses = 15 years

Capital cost for 2 fuelling maintenance facilities = $30 million

Upgrade existing facilities = $400,000

Life of fuelling facility  = 30 years

Incremental maintenance and fuel cost (per year) = $716 buses x $12,000/bus/yr = $8,592,000

Total TIP cost = $131.8 mill + $30 mill + 3 years x $8.592 mill +$400,000 =  $187.976 millions

Annualized cost = $131.8 mill / 15 years  + $30 mill / 30 years + $8.592 mill = $18.378 million

Cost Effectiveness (2005):

| NOx | = | $18.378 million/ year | = | $28,000 / ton |
| --- | --- | --- | --- | --- |
|  |  | 312 x 2.102 tons/day |  |  |

| ## M –143 Real Time Bus Schedule Information |  |
| --- | --- |
| Description: | This measure would provide real time bus schedule information to the transit riders through internet and at bus shelter display units. Satellite technology would track buses and customers would determine real-time location and arrival time of a specific bus. Local jurisdictions and state transit agencies would create integrated information system covering public transit services in the Washington region. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Cost and technology information obtained from City of Fairfax CUE buses

- Cost calculation assumes WMATA and Ride On bus GPS units are not compatible with systems providing real time schedule information.

- Capital cost spread over 3 years from 2003-2005

- Number of buses in the metropolitan region considered in the analysis for use of the Real Time bus information – 596

Alexandria Dash			49

Prince George’s county The Bus	55

Montgomery county Ryde On	237

Fairfax Connector			175

PRTC omni link			80

Total				596 Buses

Summary of Impacts (2005):

| Daily VT Reduction: | 1,188 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 17,240 | VMT |
| Daily NOx Reductions: | 0.0180 | tons/day |

Emission Analysis:

Average daily trips for the local systems				128,300

WMATA average weekday bus ridership			500,000

Average daily regional bus ridership				500,000 + 128,300 = 628,300

Total daily HBW person trips			4,400,000 (CLRP 01-06 page 19)

Total daily all person trips  =   4,400,000 x  4 	=    17,600,000  trips / region

(HBW trips are 25% of all trips)

Regional average daily bus ridership percentage

| 628,300 | = | 3.56 % |
| --- | --- | --- |
| 17,600,000 |  |  |

Estimated new transit share resulting from decrease in transit and and/or initial wait time (min).

MWCOG’s regional travel demand model uses actual headway for each route.  Since this analysis is not route specific a 5 minute savings in out of vehicle time was used.  This number is also close to estimates made in other demonstration projects.

Assumed benefit from the system = 5 minute decrease in wait time.

3.56 % → 4.66%   = 1.13% increase in bus mode share due to decrease in wait time

(source: William Allen, Mode Choice Model Sensitivity Analysis, April 1993)

For 2005 (first benefits)

Trips for 2000 – 31,156,702

Daily increase in trips on the local systems = 1.13 x 128,300 = 1450

VT reduced  = 1450 x 0.82 SOV = 1188 VT

VMT n reduced = 1188 x 14.5 = 17,240 VMT

Daily Emission Savings (2005)

Daily NOx emissions reductions:

| Cold Start | 1188 | x | 0.912 grs | x | 1 ton | = | 0.0012 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 17,240 | x | 0.980 grs | x | 1 ton | = | 0.0186 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.0198 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.0180 | tons |

Cost Analysis:

From City of Fairfax Real time bus information project.

Cost for Bus

1.	AVL/GPS System/bus		$6,995

Cost for Shelters

1.	Display units at shelters		$3,995

Total					$10,990

City of Fairfax purchased 4 display units for their 12 buses.  This TERM considers installing 172 display units in Metrorail stations and bus stops.

Cost for 596 buses	$6,550,000  cost spread over 3 years of program implementation.

Planning costs		$100,000

Bus life span		15 years

Cost  for 2005= $6,550,000/3 + $100,000 = $2,283,300

TIP Cost = $6,550,000 + $100,000 = $6,650,000

Cost Effectiveness (2005):

| NOx | = | $2,283,300 / year | = | $ 27,250 / ton |
| --- | --- | --- | --- | --- |
|  |  | 312 days x 15 yr x 0.018 tons/day |  |  |

Note:  As of July, 2001 real time bus information can be obtained through cell phones that have internet access, palm pilot units, and computers with internet access.  The cost for obtaining real time bus information to the consumer will be the cost per minute provided by the cell phone connection company.

M –143   Real Time Bus Schedule Information

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 1188 |  | 1273 |  |  |  |  |  |  |
| VMT | 17240 |  | 17471 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 1273 | x | 0.9905 grs | x | 1 ton | = | 0.00139 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 17,471 | x | 0.6995 grs | x | 1 ton | = | 0.01347 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.01486 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 1273 | x | 2.3454 grs | x | 1 ton | = | 0.00329 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 17,471 | x | 0.2717 grs | x | 1 ton | = | 0.00523 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00852 | tons |  |
| Cost effectiveness | $2,283,300 |  |  | = | 32,830 | $/ton |  |  |  |
| NOx | 312 | x | 0.01486 |  |  |  |  |  |  |
| Cost effectiveness | $2,283,300 |  |  | = | 57,260 | $/ton |  |  |  |
| VOC | 312 x 15 | x | 0.00852 |  |  |  |  |  |  |

| ## M –145 External Bicycle Racks on WMATA Buses |  |
| --- | --- |
| Description: | This measure would provide external bicycle racks in all WMATA buses. External bicycle racks provide people with an additional transportation option and as a link between transportation modes. With bike racks on buses, people can ride their bike to a bus stop and have their bike with them for the duration of the trip. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- WMATA bus fleet – 1,458 buses

- WMATA bus ridership – 600,000

Summary of Impacts (2005):

| Daily VT Reduction: | 1230 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 17835 | VMT |
| Daily NOx Reductions: | 0.01866 | tons/day |

Emission Analysis

Increase in bus ridership due to bike rack installation: 0.25% (based on the Denver, Colorado experience)

Increase in ridership: 1,500 new transit trips

VT = 1,500 x 0.82 SOV = 1,230 trips

VMT = 1,230 x 14.5 miles = 17,835miles

Daily Emission Savings (2005):

Daily NOx emissions reductions:

| Cold Start | 1230 | x | 0.912 grs | x | 1 ton | = | 0.00124 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 17,835 | x | 0.980 grs | x | 1 ton | = | 0.01927 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.02050 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.01866 | tons |

Cost Analysis:

- Cost of bike racks and installation	$1,650,000

- Planning				$100,000	(over four years)

- Advertising				$100,000	(over four years)

Total					$1,700,000

Cost Effectiveness (2005):

| NOx | = | $ 1,700,000 / year | = | $ 19,500/ ton |
| --- | --- | --- | --- | --- |
|  |  | 312 days x 15 yr x 0.01866 tons/day |  |  |

M –145 External Bicycle Racks on WMATA Buses

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 1230 |  | 1318 |  |  |  |  |  |  |
| VMT | 17835 |  | 18074 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 1318 | x | 0.9905 grs | x | 1 ton | = | 0.00144 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 18,074 | x | 0.6995 grs | x | 1 ton | = | 0.01394 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.01537 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 1318 | x | 2.3454 grs | x | 1 ton | = | 0.00341 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 18,074 | x | 0.2717 grs | x | 1 ton | = | 0.00541 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00882 | tons |  |
| Cost effectiveness | $1,700,000 |  |  | = | 23,633 | $/ton |  |  |  |
| NOx | 312 x 15 | x | 0.01537 |  |  |  |  |  |  |
| Cost effectiveness | $1,700,000 |  |  | = | 41,185 | $/ton |  |  |  |
| VOC | 312 x 15 | x | 0.00882 |  |  |  |  |  |  |

| M-146 Purchase of 100 Buses to Accommodate Ridership Growth |  |
| --- | --- |
| Description: | WMATA will purchase 100 new buses in the District of Columbia and deploy them on 36 crowded routes resulting in increased frequency. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Number of passenger per bus = 30

- Number of trips / bus = 4

- Peak frequency with additional 100 buses – 8 minutes

- 2 trips during peak period

Summary of Impacts (2005)

| Daily VT Reduction: | 9840 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 142,680 | VMT |
| Daily NOx Reductions: | 0.1493 | tons/day |

Emission Analysis

Number of passenger / bus = 30

Number of trips / bus = 4

Total number of new passenger = 100 x 30 x 4 = 12,000

New transit trips = 12,000

VT = 12,000 x 0.82 SOV = 9,840 VT

VMT = 9840 x 14.5 = 142,680 VMT

Daily Emission Savings (2005)

| Cold Start | 9840 | x | 0.912 grs | x | 1 ton | = | 0.0099 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 142,680 | x | 0.980 grs | x | 1 ton | = | 0.1541 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.1640 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.1493 | tons |

Cost Effectiveness:

| Cost Effectiveness | $33,000,000 | = $49,310 / ton |
| --- | --- | --- |
|  | 15 X 312 x 0.143 |  |

M-146  Purchase of 100 Buses to Accommodate Ridership Growth

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 9840 |  | 10540 |  |  |  |  |  |  |
| VMT | 142680 |  | 144590 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 10540 | x | 0.9905 grs | x | 1 ton | = | 0.01151 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 144,590 | x | 0.6995 grs | x | 1 ton | = | 0.11149 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.12300 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 10540 | x | 2.3454 grs | x | 1 ton | = | 0.02725 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 144,590 | x | 0.2717 grs | x | 1 ton | = | 0.04330 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.07056 | tons |  |
| Cost effectiveness | $33,000,000 |  |  | = | 39,330 | $/ton |  |  |  |
| NOx | 312 x 15 | x | 0.12300 |  |  |  |  |  |  |
| Cost effectiveness | $33,000,000 |  |  | = | 99,933 | $/ton |  |  |  |
| VOC | 312 x 15 | x | 0.07056 |  |  |  |  |  |  |

| ## M-147 Purchase Hybrid/Electric Buses in place of New Diesel Buses |  |
| --- | --- |
| Description: | This TERM funds the incremental cost of purchasing 100 hybrid electric buses as part of WMATA fleet expansion instead of clean diesel. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Purchase of 100 hybrid/electric buses in place of 100 funded in the IRP between 2003 and 2005.

- 40,000 miles per year.

- 312 days/year operation.

- Baseline emissions calculated using standard diesel running on standard diesel fuel.

- Use Mobile 6 emission factors for diesel engines and bhp-hr/mi conversion factor.

- Hybrid – Electric produce NOx emissions of 14.05 gm/mile running on ULSD as tested by CARB. (Source: Clean Diesel Technology & Hybrid Buses, New York city Transit, APTA Electric Bus Subcommittee, Seattle, Washington, October 12, 2001)

Summary of Impacts (2005)

Daily VT Reduction:		-

Daily VMT Reduction:		-

Daily NOx Reductions:		0.06 tons/day

Daily VOC Reductions:		-

Emission Analysis (2005):

Daily NOx Emissions

Diesel

| 3.90 g/bhp-hr * 4.679 bhp-hr/mi | x | 40,000 mi | x | 100 buses | = | 80.46 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 g/ton |  |  |  |  |  |  |

Hybrid/Electric buses

| 14.05 g/mi | x | 40,000 mi | x | 100 buses | = | 61.95 tons/yr. |
| --- | --- | --- | --- | --- | --- | --- |
| 907,185 g/ton |  |  |  |  |  |  |

| Diesel | - | Electric | = | ### Total |
| --- | --- | --- | --- | --- |
| 80.46 tons/yr |  | 61.95tons/yr |  | 18.51 tons/yr |

| Daily Reduction | NOx = | 18.51 tons/year | = | 0.06 tons/day |
| --- | --- | --- | --- | --- |
|  |  | 312 days |  |  |

Cost Analysis:

Incremental cost of electric bus    =   $ 91,500

Capital cost for 100 hybrid/electric buses = $9.15 mill

Life of buses = 15 years

Annualized cost = $ 9.15 mill / 15 = $610,000

Cost Effectiveness (2005):

| NOx | $610,000 | = $32,600 /ton |
| --- | --- | --- |
|  | 312 x 0.06 |  |

| ## M-148 Bus Information Displays with Maps at Increased Percentage of Stops |  |
| --- | --- |
| Description: | This measure would provide an extra 2,000 bus information boxes containing maps and schedule information. Bus information would also include schedules in other languages targeting neighborhood/areas in which other languages other than English are spoken by a large number of residents. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Decrease in waiting time = 2.5 minutes

- Average daily bus ridership  - 500,000

- Average daily local bus ridership – 124,000

- Program would be launched in 2004 and continue through 2005

Summary Impact:

| Daily VT Reduction: | 2132 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 30,914 | VMT |
| Daily NOx Reductions: | 0.0323 | tons/day |

Emission Analysis:

Local bus average daily Ridership		124,000

Average daily WMATA bus ridership	 	500,000

Average daily regional bus ridership	 	500,000 + 124,000 = 624,000

Total daily person trips		4,400,000 (conformity) x 0.25 (HBW) = 17,600,000

Regional bus mode share percentage = 624,000/17,600,000 = 3.55 %

Assumed benefit from the system = 2.5 minute decrease in wait time.

3.55 % → 4.07%   = 0.52% increase in bus mode share due to decrease in wait time

(source: William Allen, Mode Choice Model Sensitivity Analysis, April 1993)

WMATA Buses – 500,000 (0.52%) = 2600 new trips

VT reductions       = 2600 x 0.82 SOV = 2132

VMT reductions    = 2132 x 14.5 miles = 30914

Daily NOx Emission Reductions (2005):

| Cold Start | 2132 | x | 0.912 grs | x | 1 ton | = | 0.0021 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 30,914 | x | 0.980 grs | x | 1 ton | = | 0.0334 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.0355 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.0323 | tons |

# Cost Analysis:

Cost per box = $120.00

Number of boxes = 2000

Cost for 2000 display boxes = $240,000

Capital cost per car – $30,000

Operating Cost for car to change schedule = $40,000

Life of car – 6 years

Annualized cost – $240,000 / 3 years + $30,000/6 years + $40,000 = $125,000

TIP Cost: $240,000 + $30,000 + $40,000 x 2 = $350,000

Cost Effectiveness (2005):

| NOx | = | $ 125,000 | = | $12,520/ ton |
| --- | --- | --- | --- | --- |
|  |  | 312 days x 0.032 tons |  |  |

M-148  Bus Information Displays with Maps at Increased Percentage of Stops

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 2132 |  | 2284 |  |  |  |  |  |  |
| VMT | 30914 |  | 31328 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 2284 | x | 0.9905 grs | x | 1 ton | = | 0.00249 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 31,328 | x | 0.6995 grs | x | 1 ton | = | 0.02416 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.02665 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 2284 | x | 2.3454 grs | x | 1 ton | = | 0.00590 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 31,328 | x | 0.2717 grs | x | 1 ton | = | 0.00938 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.01529 | tons |  |
| Cost effectiveness | $125,000 |  |  | = | 15034 | $/ton |  |  |  |
| NOx | 312 | x | 0.02665 |  |  |  |  |  |  |
| Cost effectiveness | $125,000 |  |  | = | 26208 | $/ton |  |  |  |
| VOC | 312 | x | 0.01529 |  |  |  |  |  |  |

| ## M-149 Automatic Vehicle Locator for Buses (AVL) |  |
| --- | --- |
| Description: | This measure would provide bus location information to bus dispatchers in the WMATA system. Tracking buses and timing headways would decrease wait time for commuters and improve arrival/departure time for buses across the region. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Total number of WMATA buses = 1,458

- Average daily WMATA bus ridership = 500,000 trips/day

- WMATA Planning Staff estimate a 1.5% increase in bus ridership

Summary of Impacts (2005):

| Daily VT Reduction: | 6150 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 89,175 | VMT |
| Daily NOx Reductions: | 0.0933 | tons/day |

Emission Analysis:

Average daily increase in trips on the five systems = 500,000 x 1.5% = 7,500

New transit trips = 7,500

VT reduced = 7500 x 0.82 SOV = 6150 VT

VMT reduced = 6150 x 14.5 = 89175 VMT

Daily NOx Emission Reductions (2005):

| Cold Start | 6150 | x | 0.912 grs | x | 1 ton | = | 0.0062 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 89,175 | x | 0.980 grs | x | 1 ton | = | 0.0963 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.1025 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.0933 | tons |

Cost Analysis:

Some components of an AVL system will be installed as part of an on-going radio system upgrade.

WMATA estimates that it would need an additional $10,000,000 to complete installation of an AVL system on their entire fleet.

Cost Effectiveness (2005):

| NOx | = | $ 10,000,000 | = | $ 22,900 / ton |
| --- | --- | --- | --- | --- |
|  |  | 312 days x 15 yr x 0.0933 tons |  |  |

M-149  Automatic Vehicle Locator for Buses (AVL)

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 6150 |  | 6588 |  |  |  |  |  |  |
| VMT | 89175 |  | 90369 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 6588 | x | 0.9905 grs | x | 1 ton | = | 0.00719 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 90,369 | x | 0.6995 grs | x | 1 ton | = | 0.06968 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.07687 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 6588 | x | 2.3454 grs | x | 1 ton | = | 0.01703 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 90,369 | x | 0.2717 grs | x | 1 ton | = | 0.02707 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.04410 | tons |  |
| Cost effectiveness | $10,000,000 |  |  | = | 27,800 | $/ton |  |  |  |
| NOx | 312 x 15 | x | 0.07687 |  |  |  |  |  |  |
| Cost effectiveness | $10,000,000 |  |  | = | 48,450 | $/ton |  |  |  |
| VOC | 312 x 15 | x | 0.04410 |  |  |  |  |  |  |

| ## M-150 Enhanced Commuter Services On Major Corridors (HOV Facilities) |  |
| --- | --- |
| Description: | This TERM proposes bus service on corridors with HOV facilities and bus lanes such as US 50, I-270, and US 29. Commuters would be picked up at Metrorail Park & Ride facilities close to Metro stations and transported to major work centers. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- US 50 - Double existing rush hour service on B21,22; B29; and C28 connecting Bowie Park Park & Ride at MD Route 197 with New Carrollton Station.  Establish: 15 minute headway non-stop rush only express route between new Prince George’s Stadium Park and Ride on Route 301 and New Carrollton Station.

- I-270 – New: 15 minute headway rush only express between Urbana Park and Ride and Shady Grove Station via I-270, MD Route 124 Park & Ride and Sam Eig Highway.

- US-29 – New: 10 minute headway rush  only express linking Burtonsville, Greencastle , and Briggs Chaney Park & Ride lots with Silver Spring Metrorail station.

Summary of Impacts (2005):

Daily VT Reduction:		-

Daily VMT Reduction:		49,509 VMT

Daily NOx Reductions:		0.04866 tons/day

Daily VOC Reductions:		0.018369 tons/day

Emission Impacts for (2005):

For this measure there will be no Cold Start and hot Soak reductions since commuters would continue to drive to Park and Ride parking spaces.

Daily NOx emission reductions (2005):

| Running | 49,509 | x | 0.980 grs | x | 1 ton | = | 0.053483 tons/day |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907, 185 grs |  |  |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.04866 tons/day |

Daily VOC emission reductions (2005)

| Running | 49,509 | x | 0.347 grs | x | 1 ton | = | 0.01893 tons/day |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907, 185 grs |  |  |
|  |  |  |  | Tier 2 Controls (0.97) |  |  | 0.01836 tons/day |

Cost Analysis:

| ### POLLUTION MITIGATION SERVICES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Annual Cost | Annual Revenue | Annual Subsidy | Annual Passengers | Number of Buses |
| US 50 | 1,068,000 | 175,000 | 893,000 | 313,000 | 11 |
| I 270 | 814,000 | 204,000 | 610,000 | 364,000 | 6 |
| US 29 | 814,000 | 204,000 | 610,000 | 364,000 | 8 |

US-50

Cost per bus - $330,000

Cost of Fleet - $330,000 x 11 buses = $3,630,000

Annual cost = (3,630,000/15 years) + 893,000 (annual subsidy)

Total = $1,135,000

# I-270

Cost per bus - $330,000

Cost of fleet - $330,000 x 6 = $1,980,000

Annual cost = ($1,980,000/15 years) + $610,000 (annual subsidy)

Total  = $742,000

# US-29

Cost per bus - $330,000

Cst of fleet - $330,000 x 8 = $2,640,000

Annual cost = ($2,640,000/15 years) + $610,000 (annual subsidy)

Total = $786,000

Total annual cost for the three routes = $2,663,000

Cost Effectiveness (2005):

| NOx | = | $2,663,000 /yr | = | $ 218,900 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 yr x 0.04866 |  |  |

| VOC | = | $2,663,000 /yr | = | $ 579,890 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 yr x 0.01836 |  |  |

M-150 Enhanced Commuter Services On Major Corridors (HOV Facilities)

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 0 |  | 0 |  |  |  |  |  |  |
| VMT | 49509 |  | 50172 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 0 | x | 0.9905 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 50,172 | x | 0.6995 grs | x | 1 ton | = | 0.03869 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.03869 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 0 | x | 2.3454 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 50,172 | x | 0.2717 grs | x | 1 ton | = | 0.01503 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.01503 | tons |  |
| Cost effectiveness | $2,663,000 |  |  | = | 220630 | $/ton |  |  |  |
| NOx | 312 | x | 0.03869 |  |  |  |  |  |  |
| Cost effectiveness | $2,663,000 |  |  | = | 568018 | $/ton |  |  |  |
| VOC | 312 | x | 0.01503 |  |  |  |  |  |  |

| ## M-151 Enhanced Commuter Services On Major Corridors (Reverse Commute) |  |
| --- | --- |
| Description: | This TERM proposes bus service to Potomac Mills and Arundel Mills shopping centers from Metrorail stations. The service would benefit reverse commuters whose work place is in Prince William and Anne Arundel counties. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Potomac Mills – Route: from Huntington station via North Kings highway, Route 1, Telegraph Road, Whernside St., Pohick Rd., Route 1, I-95 (at exit 161), Potomac Mills Road and return.

- Arundel Mills – New two-way express between Anacostia Station and Arundel Mills mall via Deanwood Station and Baltimore-Washington Parkway.

- This measure would benefit from Cold Start and Hot Soak emissions since commuters would be travelling on Metrorail and continuing their trip in the bus service to shopping malls.

Summary Impacts (2005):

Daily VT Reduction:		1,950 VT

Daily VMT Reduction:		49,509 VMT

Daily NOx Reductions:		0.0504 tons/day

Daily VOC Reductions:		0.023 tons/day

Emission Impacts for (2005):

Daily NOx emission reductions (2005):

| Cold Start | 1,950 | x | 0.912 grs | x | 1 ton | = | 0.001964 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |

| Running | 49,509 | x | 0.980 grs | x | 1 ton | = | 0.03062 tons/day |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907, 185 grs |  |  |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.0504 tons/day |

Daily VOC emission reductions (2005)

| Cold Start + Hot Soak | 1,950 | x | 2.287 grs | x | 1 ton | = | 0.004926 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 trip |  | 907,185 grs |  |  |

| Running | 49,509 | x | 0.347 grs | x | 1 ton | = | 0.0189 tons/day |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907, 185 grs |  |  |
|  |  |  |  | Tier 2 Controls (0.97) |  |  | 0.023 tons/day |

Cost Analysis:

| ### POLLUTION MITIGATION SERVICES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Annual Cost | Annual Revenue | Annual Subsidy | Annual Passengers | Number of Buses |
| Arundel Mills | 1,160,000 | 174,000 | 986,000 | 171,000 | 4 |
| Potomac Mills | 1,443,000 | 358,400 | 1,311,000 | 426,700 | 6 |

Arundel Mills

Cost per bus - $330,000

Cost of Fleet - $330,000 x 4 buses = $1,320,000

Annual cost = (1,320,000/15 years) + 986,000 (annual subsidy)

Total = $1,074,000

# Potomac Mills

Cost per bus - $330,000

Cost of fleet - $330,000 x 6 = $1,980,000

Annual cost = ($1,980,000/15 years) + $1,311,000 (annual subsidy)

Total  = $1,443,000

Total annual cost for the three routes = $2,517,000

Cost Effectiveness (2005):

| NOx | = | $2,517,000 | = | $ 199,762 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 yr x 0.0504 |  |  |

| VOC | = | $2,517,000 | = | $ 437,739 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 yr x 0.023 |  |  |

M-151 Enhanced Commuter Services On Major Corridors (Reverse Commute)

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 1950 |  | 2089 |  |  |  |  |  |  |
| VMT | 49509 |  | 50172 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 2089 | x | 0.9905 grs | x | 1 ton | = | 0.00228 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 50,172 | x | 0.6995 grs | x | 1 ton | = | 0.03869 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.04097 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 2089 | x | 2.3454 grs | x | 1 ton | = | 0.00540 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 50,172 | x | 0.2717 grs | x | 1 ton | = | 0.01503 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.02043 | tons |  |
| Cost effectiveness | $2,517,000 |  |  | = | 196925 | $/ton |  |  |  |
| NOx | 312 | x | 0.04097 |  |  |  |  |  |  |
| Cost effectiveness | $2,517,000 |  |  | = | 394940 | $/ton |  |  |  |
| VOC | 312 | x | 0.02043 |  |  |  |  |  |  |

| ## M-152 Enhanced Commuter Services On Major Corridors (Rail Relief) |  |
| --- | --- |
| Description: | This TERM proposes express bus service from Reston/Herndon, Centreville, and Springfield to Pentagon and downtown Washington, D.C. |
| Analysis Tool: | Sketch Planning |

Summary of Impacts (2005):

Daily VT Reduction:		4,715 VT

Daily VMT Reduction:		68,367 VMT

Daily NOx Reductions:		0.067208 tons/day

Daily VOC Reductions:		0.025366 tons/day

Emission Impacts for (2005):

Daily NOx emission reductions (2005):

| Running | 68,367 | x | 0.980 grs | x | 1 ton | = | 0.073855 tons/day |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907, 185 grs |  |  |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.067208 tons/day |

Daily VOC emission reductions (2005)

| Running | 68,367 | x | 0.347 grs | x | 1 ton | = | 0.026151 tons/day |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907, 185 grs |  |  |
|  |  |  |  | Tier 2 Controls (0.97) |  |  | 0.02536 tons/day |

Cost Analysis

I-66 Reston/Herndon and Centreville

Annual cost = ($18,600,000/15 years) + 5,700,000 (annual subsidy)

Total = $6,940,000

# I-395 Springfield

Annual cost = ($3,000,000/15 years) + $900,000 (annual subsidy)

Total  = $1,100,000

Total annual cost for the three routes = $8,040,000

Cost Effectiveness (2005):

| NOx | = | $8,040,000 | = | $ 478,514 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 yr x 0.067208 |  |  |

| VOC | = | $8,040,000 | = | $ 1,267,838 / ton |
| --- | --- | --- | --- | --- |
|  |  | 250 x 1 yr x 0.025366 |  |  |

M-152 Enhanced Commuter Services On Major Corridors (Rail Relief)

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 0 |  | 0 |  |  |  |  |  |  |
| VMT | 68367 |  | 69282 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 0 | x | 0.9905 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 69,282 | x | 0.6995 grs | x | 1 ton | = | 0.05342 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.05342 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 0 | x | 2.3454 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 69,282 | x | 0.2717 grs | x | 1 ton | = | 0.02075 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.02075 | tons |  |
| Cost effectiveness | $8,040,000 |  |  | = | 482377 | $/ton |  |  |  |
| NOx | 312 | x | 0.05342 |  |  |  |  |  |  |
| Cost effectiveness | $8,040,000 |  |  | = | 1241895 | $/ton |  |  |  |
| VOC | 312 | x | 0.02075 |  |  |  |  |  |  |

| ## M-154 Extend Peak Service Period |  |
| --- | --- |
| Description: | This measure would extend peak period service on the Metrorail system to the shoulders of the peak period. During the peak, metro rail run 6-minute frequencies on each line (on average), which results in 6 min. frequencies at the edges of the system, and 3-minute frequencies in the core (where the lines overlap.) Off peak, the frequencies are 12 and 6 minutes. The additional "peak" service" will be between 9-11 AM and 3-4 PM and 7-8 PM. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Peak period: 6-9 AM and 4-7 PM

- Peak Period ridership – 440,000

- WMATA estimates 1.5% increase in ridership during the period

Summary of Impact:

| Daily VT Reduction: | 5410 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 78,445 | VMT |
| Daily NOx Reductions: | 0.0821 | tons/day |

Emission Analysis:

WMATA ridership increase = 1.5 %

New riders during the period  = 6600

Daily increase in trips = 6600

VT reduced  = 6600 * 0.82 = 5410

VMT  reduced = 5410 x 14.5 = 78445 VMT

Daily Emission Savings (2005)

| Cold Start | 5410 | x | 0.912 grs | x | 1 ton | = | 0.0054 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 78,445 | x | 0.980 grs | x | 1 ton | = | 0.0847 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.0902 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.0821 | tons |

Cost Effectiveness:

Annual operating cost of the program = $10,000,000

| NOx = | 10,000,000 |  | = $390,400 / ton |
| --- | --- | --- | --- |
|  | 312 x 0.0821 |  |  |

M-154 Extend Peak Service Period

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 5410 |  | 5795 |  |  |  |  |  |  |
| VMT | 78445 |  | 79495 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 5795 | x | 0.9905 grs | x | 1 ton | = | 0.00633 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 79,495 | x | 0.6995 grs | x | 1 ton | = | 0.06130 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.06762 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 5795 | x | 2.3454 grs | x | 1 ton | = | 0.01498 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 79,495 | x | 0.2717 grs | x | 1 ton | = | 0.02381 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.03879 | tons |  |
| Cost effectiveness | $10,000,000 |  |  | = | 473967 | $/ton |  |  |  |
| NOx | 312 | x | 0.06762 |  |  |  |  |  |  |
| Cost effectiveness | $10,000,000 |  |  | = | 826255 | $/ton |  |  |  |
| VOC | 312 | x | 0.03879 |  |  |  |  |  |  |

| ## M-155 Expansion of Car Sharing Program |  |
| --- | --- |
| Description: | This TERM funds incentives for 1000 new car sharing customers. Car sharing customers typically increase their transit ridership and decrease driving. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- The program funds 1000 new members

- Approximately 35 members share one car

- Every car generates 5 persons switching to transit.

- For 1000 users the program will need 29 cars

Summary of Impacts (2005)

Daily VT Reduction:		290 VT

Daily VMT Reduction:		4205 VMT

Daily NOx Reductions:		0.004134 tons/day

Daily VOC Reductions:		0.001560 tons/day

Emission Analysis

Every car generates 5 transit trips

1 car = 35 members

1000 users = 29 cars

29 cars x 5 transit trips = 145 transit trips

145 x 2 (round trip) = 290 VT

VMT = 290 x 14.5 miles = 4205

Daily Emission Savings (2005)

Daily NOx emissions reductions:

| Running | 4205 | x | 0.980 grs | x | 1 ton | = | 0.004543 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 |  |  |
| Total NOx emissions reduction with Tier 2 Controls (0.91) |  |  |  |  |  |  | 0.004134 tons |

Daily VOC emissions reductions:

| Running | 4205 | x | 0.347 grs | x | 1 ton | = | 0.001608 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |
|  | Total VOC emissions reduction with Tier 2 Controls (0.97) |  |  |  |  |  | 0.001560 tons |

Cost Analysis

Subsidy = $125.00 x 1000 users = $125,000

Administrative cost = $50,000/yr

Annual Cost = $175,000 per year

Total TIP cost (2002-04):    ($125 x 1000  + 50,000) x 3 years   =   $525,000

Cost Effectiveness (2005)

| NOx | = | $ 175,000 | = | $ 135,700 / ton |
| --- | --- | --- | --- | --- |
|  |  | 312 days x 1 yr x 0.004134 tons |  |  |

| VOC | = | $ 175,000 | = | $ 373,930 / ton |
| --- | --- | --- | --- | --- |
|  |  | 312 days x 1 yr x 0.0015 tons |  |  |

M-155 Expansion of Car Sharing Program

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 0 |  | 0 |  |  |  |  |  |  |
| VMT | 4205 |  | 4261 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 0 | x | 0.9905 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 4,261 | x | 0.6995 grs | x | 1 ton | = | 0.00329 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00329 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 0 | x | 2.3454 grs | x | 1 ton | = | 0.00000 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 4,261 | x | 0.2717 grs | x | 1 ton | = | 0.00128 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00128 | tons |  |
| Cost effectiveness | $175,000 |  |  | = | 170706 | $/ton |  |  |  |
| NOx | 312 | x | 0.00329 |  |  |  |  |  |  |
| Cost effectiveness | $175,000 |  |  | = | 439488 | $/ton |  |  |  |
| VOC | 312 | x | 0.00128 |  |  |  |  |  |  |

| ## M-156 Free Bus-to-Rail/Rail-to Bus Transfer (Similar to NYC Pricing Structure) |  |
| --- | --- |
| Description: | This program would institute a free bus to rail transfer similar to the reduced fare rail to bus transfer. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- 3% increase in bus/rail ridership based on WMATA planning staff.

- Current bus to rail transfer -  200,000

Summary of Impacts (2005):

| Daily VT Reduction: | 4,920 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 71,340 | VMT |
| Daily NOx Reductions: | 0.0746 | tons/day |

Emission Analysis:

New ridership  		 200,000 x 0.03  =  6,000

VT reduced  		 6000 x 0.82  =  4,920

VMT reduced 		 4920 x 14.5  =  71,340

Daily NOx Emissions Reductions:

| Cold Start | 4920 | x | 0.912 grs | x | 1 ton | = | 0.0049 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 71340 | x | 0.980 grs | x | 1 ton | = | 0.0771 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.0820 | tons |
| Total emission reductions with Tier 2 Control (0.91) |  |  |  |  |  |  | 0.0746 | tons |

Cost Analysis:

Loss in revenue: $0.59 x 200,000 riders x 312 days = $36.816 millions

Cost Effectiveness:

| NOx = |  |  |  |
| --- | --- | --- | --- |
|  | $36.816 millions |  | = $1.58 millions/ton |
|  | 312 x 0.0746 |  |  |

M-156  Free Bus-to-Rail/Rail-to Bus Transfer (Similar to NYC Pricing Structure)

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 4920 |  | 5270 |  |  |  |  |  |  |
| VMT | 71340 |  | 72295 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 5270 | x | 0.9905 grs | x | 1 ton | = | 0.00575 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 72,295 | x | 0.6995 grs | x | 1 ton | = | 0.05574 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.06150 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 5270 | x | 2.3454 grs | x | 1 ton | = | 0.01363 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 72,295 | x | 0.2717 grs | x | 1 ton | = | 0.02165 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.03528 | tons |  |
| Cost effectiveness | $36,816,000 |  |  | = | 1918743 | $/ton |  |  |  |
| NOx | 312 | x | 0.06150 |  |  |  |  |  |  |
| Cost effectiveness | $36,816,000 |  |  | = | 3344900 | $/ton |  |  |  |
| VOC | 312 | x | 0.03528 |  |  |  |  |  |  |

| ## M -158 Free Bus Service Off-Peak (10:00 AM –2:00 PM Mid-Day and Weekends) |  |
| --- | --- |
| Description: | Free bus service off-peak (10-2 mid-day, weekends): Free service during the mid day and all day on weekends. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Fare discount - 56 cent

- Total daily weekday ridership in off-peak period is 192,800

- Total weekend ridership  -  171,800

- This measure is anticipated to attract 5040 new transit riders daily as estimated by WMATA planning staff.

- Total annual cost of this measure is $21,821,184

Summary of Impacts (2005):

| Daily VT Reduction: | 4133 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 59,926 | VMT |
| Daily NOx Reductions: | 0.0627 | tons/day |

Emission Analysis:

Weekend bus ridership – 192,000

Week-end off-peak – 171,800

Average off peak and weekend = 181,900

Average fare discount = 56 cents ( 27 cents 1980 dollar value)

Net increase in ridership estimated by WMATA = 5040

VT reduced:	   		5040 x 0.82 = 4,133

VMT  reduced:			4133 x 14.5  = 59,926

NOx Emission Reductions:

| Cold Start | 4133 | x | 0.912 grs | x | 1 ton | = | 0.0042 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |

| Running | 59,926 | x | 0.980 grs | x | 1 ton | = | 0.0647 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.0689 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.0627 | tons |

Cost Effectiveness:

| NOx = | 21,821,000 | = $1,115,456 / ton |
| --- | --- | --- |
|  | 312 day x 0.0627 |  |

M -158 Free Bus Service Off-Peak (10:00 AM –2:00 PM Mid-Day and Weekends)

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 4133 |  | 4427 |  |  |  |  |  |  |
| VMT | 59926 |  | 60728 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 4427 | x | 0.9905 grs | x | 1 ton | = | 0.00483 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 60,728 | x | 0.6995 grs | x | 1 ton | = | 0.04683 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.05166 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 4427 | x | 2.3454 grs | x | 1 ton | = | 0.01145 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 60,728 | x | 0.2717 grs | x | 1 ton | = | 0.01819 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.02963 | tons |  |
| Cost effectiveness | $21,821,000 |  |  | = | 1353851 | $/ton |  |  |  |
| NOx | 312 | x | 0.05166 |  |  |  |  |  |  |
| Cost effectiveness | $21,821,000 |  |  | = | 2360109 | $/ton |  |  |  |
| VOC | 312 | x | 0.02963 |  |  |  |  |  |  |

| ## M- 159 W15-590 - Diesel Fuel Additive |  |
| --- | --- |
| Description: | This measure represents the use of a fuel additive, W15-590, in vehicles to reduce NOx . This additive can also be blended with the fuel before delivery from the distribution center. It has shown a 10.81% reduction in NOx in lab tests. The company says the results are much higher in a “real world” situation. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- It has shown a 10.81% reduction in NOx in lab tests.  The company says the results are much higher in a “real world” situation.

- This additive can also be blended with the fuel before delivery from the distribution center.

- This measure represents the use of a fuel additive, W15-590, in vehicles to reduce NOx .

- 500 buses will participate in the program.

- W15-590 will reduce NOx by 10.81%. (Based on the lab results presented by the manufacturer)

- Average cost of fuel additive:  4 cents/gallon

- Even though manufacturer, Woodrow International Limited claims savings in fuel expenses due to improved fuel economy, this analysis does not account for savings.

- Average age of bus in program will be 7 years.

Summary of Impacts (2005):

Daily NOx reductions (minimum): 0.133 tons/day

Emissions Analysis:

Number of buses using W15-590:   500

NOx emissions factor for buses over 7 years: 15.87 gm/mile

Average bus mileage:   140 miles/day/bus

# Bus VMT:  140 miles/day x 500= 70,000 miles/day

Reduction in NOx emission (minimum):  10.81% x 15.87 gm/mile = 1.72 gm/mile

NOx reduction for 500 buses per day (minimum): 1.72 gm/mile x 70,000 miles/907185 =0.133 tons/day

Cost Analysis:

Bus fuel consumption:  2 miles/gallon

Bus fuel consumption per day: 70 gallons/day/bus

Fuel consumption for 500 buses:  70 gallons/day/bus x 500= 35,000 gallons/day

Cost of W15-590 per treated gallon:  4.5 cents

Fuel expenditure reduction:  6%

Cost of fuel:  $.70 per gallon

Cost for 500 buses per day with W15-590:  35,000 x 4.5 cents/gallon = $1,575/day

Fuel expenditure reduction:  35,000 x 6% = 2,100 x $.70= $1,470/day

Net cost for reducing NOx:  $1,575 - $1,470 = $105/day

Administrative Costs = $160,000 for FY 2004 and FY 2005

Net Annual Fuel Cost = 32,760

Total TIP Cost = $160,000 + 32,760 = $225,520

Cost Effectiveness:

| NOx | = | $112,760 | = | $2,700 / ton |
| --- | --- | --- | --- | --- |
|  |  | 312 days x 0.133 |  |  |

| ## M-160 Bose Automobile Anti-Air Pollutant and Energy Conservation System |  |
| --- | --- |
| Description: | The Bose Automobile Anti- Air Pollutant and Energy Conservation System is a mechanical, gas turbine operated system with no platinum catalysts involved as in catalytic converter systems. This system can be used with all types of fuel. It is expected to achieve 15% fuel economy and reduces NOx emissions by 50% as claimed by the manufacturer. In this program 500 buses would be retrofitted with the Bose Energy Conversion System. |
| Analysis Tool: | Sketch Planning |

About  Bose Energy Conservation System:

The Bose Energy Conservation System is developed for high-speed, centrifugal separation of the lighter pollutant combustible gases in the automobile exhaust gas stream from the heavier non-combustible gases. This system uses a high-speed free spinning turbine attached to the auto exhaust system. By spinning around and creating centrifugal force, the device reduces gaseous emissions and recycles combustible gases to the carburetors, resulting in potential energy saving and reduction in the pollutant emissions. It is a mechanical, gas turbine operated system with no platinum catalysts involved as in catalytic converter systems. The Bose system can be used with all types of fuel namely leaded, unleaded gasoline, diesel or alternate fuel. It is expected to achieve 15 % fuel economy and will cost about $525 per vehicle. The Bose system was tested at the Gulf Research Laboratory in Pittsburgh, PA. The test result at the Gulf Research Laboratory shows lifecycle pollutant emissions as 900,000 grams for the Bose system verses 2,621,488 grams for a typical converter, reducing all pollutants by 66%. For NOx the reduction is 50%.

Assumptions:

- 500 buses will participate in the program.

- The Energy Conservation System will reduce NOx by 50%. (As claimed by Bose Research and Development. Inc.)

- Cost of the energy conversion system: $525 per unit

- Average age of bus in the program will be 7 years.

Summary of Impacts (2005):

Daily NOx Reductions:   0.61 tons/day

Emission Analysis:

Number of buses retrofitted with the system: 500

NOx emission factor for buses over 7 years: 15.87 gm/mile

Average bus mileage: 140 miles/day/bus

Total Bus VMT : 140 miles/day x 500 = 70,000 miles/day

Reduction in the NOx emission: 50% x 15.87 gm/mile = 7.9 gm/mile

NOx reduction for 500 buses per day = 7.9 gm/mile x 70,000 miles/ 907,185 = 0.61 tons/day

Cost Analysis:

Cost of energy conversion device: $525

Capital Cost of 500 devices to be used in 500 buses: $525 x 500 =  $262,500

Life span: 5 years

Number of day’s buses operated in a year: 312

Administrative Costs = $160,000 (FY 2004 & 2005)

Maintenance Cost = ($100 per device / year x 500) = $50,000

Total TIP Cost = $472,000

Cost Effectiveness:

| NOx = | $182,500 | = $1,000/ton |
| --- | --- | --- |
|  | 312 days x 0.61 tons/day |  |

| ## EPA Candidate Measure # 1 ## M-161 Diesel Emulsion Fuel Additive (Non-road or Highway) |  |
| --- | --- |
| Description: | This program will introduce a fuel additive to diesel to reduce NOx and other pollutants. Both on-road and off-road vehicles would participate in the program. |
| Analysis Tool: | Sketch Planning (By EPA) |

Assumptions:

- 500 vehicle (on-road and off-road) will participate in the program.

- Cost of fuel additive:  $1,200/vehicle per year

- The program will be implemented  in FY 2004 and 2005 (250 vehicles/year)

- Administrative expenses = $80,000/year

- The program will pay participants two years of fuel additive costs

Summary of Impacts (2005):

Daily NOx Reductions:     0.18  tons/day

Emission Analysis:

Daily NOx Reductions:     0.18  tons/day

(As estimated by EPA)

Cost Estimation:

Cost of fuel additive:   $1,200/vehicle/year

Total fuel additive cost:  $1,200 x 500 x 2  =  $1,200,000

Administrative cost = $160,000

Total TIP Period Cost = $1,360,000

Cost Effectiveness:

Annualized Cost = $680,000

| NOx | $680,000 | = | $ 12,100 / ton |
| --- | --- | --- | --- |
|  | 312 x 0.18 |  |  |

| ## EPA Candidate Measure # 2 ## M-162 Early Engine Retirement (Pre-88) |  |
| --- | --- |
| Description: | This is a voluntary program designed to install new diesel engines by replacing old high polluting diesel engines in private vehicles. This is similar to “cash for clunkers” where instead of cash, the operator would get a new diesel engine. |
| Analysis Tool: | Sketch Planning (US EPA) |

Assumptions:

- 500 old trucks will be replaced through program

- Average truck operation: 8 hrs/day, 300 days/yr.

- Cost of one engine including installation:   $8000

- The program will be implemented over a two year period FY 2004 and 2005

- The program will provide for maintenance at $200 / year / engine for three years (three year warranty)

Summary of Impacts (2005):

Daily NOx Reductions:     0.9  tons/day

Emission Analysis:

Daily NOx Reductions:     0.9  tons/day

(As estimated by EPA)

Cost Estimation:

Cost of one engine:   $8,000

Maintenance cost: $200/year/engine

Life span: 10 years

Capital Cost of 500 engines:   $ 4 million

Two year administrative costs = $160,000

Maintenance cost per year: $200 x 500 = $100,000

Total TIP Cost = $4,000,000 + $300,000 + $160,000 = $4,460,000

Annualized Cost:  $4 million / 10 years + $100,000 + $80,000= $580,000

Cost Effectiveness:

| NOx = | $580,000 | = $2,200/ton |
| --- | --- | --- |
|  | 300 days x 0.9 tons/day |  |

| ## EPA Candidate Measure # 3 ## M-163 Truck Idling (Truck Stops and Auxiliary Power Unit ) |  |
| --- | --- |
| Description: | This is a voluntary program designed to install pollution-reduction technology on existing diesel vehicles and equipment. Under this program it is proposed to use a small diesel auxiliary power unit (APU), which will be mounted on the truck chassis to provide heat, air conditioning, or engine heat in lieu of operating the engine. This is expected to reduce the vehicle’s NOx idling emission by 90%. |
| Analysis Tool: | Sketch Planning (USEPA) |

Assumptions:

- Typical truck engine emits 0.3 tons/year of NOx while idling.

- Average truck operation: 8 hrs/day, 312 days/yr.

- Cost of one APU + installation cost:   $5,000 + $1,000 = $6,000

- Maintenance cost: $550/year/APU

- Users will pay for operating cost.

- The program will be administrated over a two year period from FY 2004 through FY 2005.

- The program will pay for the maintenance of the system during the first year.

Summary of Impacts (2005):

Daily NOx Reductions:     0.4  tons/day

## Emission Analysis:

Daily NOx Reductions:     0.4  tons/day

(As estimated by EPA: refer to Document “Voluntary Diesel Retrofit Program”)

Cost Estimation:

Cost of one engine and Installation:   $5,000 + $1,000 = $6,000

Maintenance cost: $550/year/APU

Life span: 10 years

Capital Cost of 500 engines:   $3 million

Maintenance cost per year: $550  x 500 = $275,000

Two years administrative cost = 2 x $80,000 = $160,000

Annualized Cost:  $3 million / 10 years + $275,000 = $575,000

TIP cost: $3,000,000 + $160,000 + $275,000 = $3,710,000

Cost Effectiveness:

|  | NOx = |  | $575,000 | = $4,600/ton |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 312 days x 0.4 tons/day |  |  |
| ## EPA Candidate Measure # 4 ## M-164 International Green Diesel Retrofit |  |  |  |  |  |
| Description: |  | Under this program 500 transit buses would be fitted with a quad catalytic filter and Ultra Low Sulfur Diesel fuel would be used. |  |  |  |
| Analysis Tool: |  | Sketch Planning (US EPA) |  |  |  |

Assumptions:

- 500 buses will be used in program at 250 per year starting in FY 2004 and completed by FY 2005

- Average bus operation: 8 hrs/day, 312 days/yr

- Cost per filter:   $8000

- Maintenance cost: $1,000 /year/engine

- Cost per gallon of Ultra Low Sulfur Diesel fuel: 15 cents

- Administrative costs = $80,000/yr for two years

Summary of Impacts (2005)

Daily NOx Reductions:     0.14 tons/day for 500 buses

Daily Emission Savings (2005)

NOx Reductions  0.14 tons/day for 500 buses

Cost Estimation:

Cost per filter:   $8,000

Administrative costs = $160,000 (FY 2004-2005)

Operating and maintenance cost: $1,000/year

Life span: 15 years

Capital Cost of 500 engines:   $ 4 million

Maintenance cost for one year: $1,000 * 500 = $500,000

Cost of fuel per bus per year: 20,000 gallons x 15 cent per gallon = $3,000/bus

Cost of fuel for 500 buses per year: $3,000 x 500 = $1,500,000

Annualized Cost:  $4 million / 15 years + $500,000 + $1,500,000 = $2,266,666

Total TIP cost 	= $4,000,000 + $500,000 + $1,500,000 + $600,000

= $6,160,000

Cost Effectiveness:

| NOx = | $6,160,000 | = $141,000/ton |
| --- | --- | --- |
|  | 312 days x 0.14 tons/day |  |

| ## M-165 Bike Stations at Rail Station |  |
| --- | --- |
| Description: | This measure assumes construction of a Bike Station at the Silver Spring Transit Station. A Bike Station is a full-service bicycle storage and rental facility with guarded valet bicycle parking, bicycle rentals, repairs, and supplies. |
| Analysis Tool: | Sketch Planning |

Assumptions:

- Used 2% growth rate, compounded annually, (from 2001 to 2006 transit work trip growth rate in FY2001-2006 TIP / CLRP air quality conformity analysis), to grow new users to 2005 figures.

- Average trip length = 14.5 miles

- Capital cost = $200,000 (will vary by location)

- Operating cost includes two attendants and cost for leasing station space.

- E missions and cost analysis shown here is for Silver Spring station only.

- 5 additional locations would be selected in consultation with local authorities and rail transit providers.

- 10 year life-span for cost analysis

- 85 percent of new users drive alone prior to using transit.

Summary of Impacts (2005)

| Daily VT Reduction: | 107 | VT |
| --- | --- | --- |
| Daily VMT Reduction: | 1552 | VMT |
| Daily NOx Reductions: | 0.00162 | tons/day |

Emission Analysis (2005)

Current average workday users at Palo Alto Bike Station = 88

Current Silver Spring Metrorail station bike rack users= 30

Number of new users = 58

New users in 2005 = 58 x (1.02)4 = 63

Number of new trips = 63 x 2 = 126

Number of SOV trips = 126 x 0.85 = 107

VMT = 107 x 14.5 = 1552

Daily Emissions Reductions:

| Cold Start | 107 | x | 0.912 grs | x | 1 ton | = | 0.00011 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 1,552 | x | 0.980 grs | x | 1 ton | = | 0.00168 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.00178 | tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.00162 | tons |

VOC Savings:

| Cold Start + Hot Soak | 107 | x | 2.287 grs | x | 1 ton | = | 0.00027 | tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |
| Running | 1,552 | x | 0.347 grs | x | 1 ton | = | 0.0006 | tons |
|  |  |  | 1 mi |  | 907,185 |  |  |  |
|  |  |  |  |  | Total |  | 0.00086 | tons |
|  |  |  |  | Tier 2 Controls (0.97) |  |  | 0.00083 | tons |

Cost analysis:

Capital cost: $200,000

Operating cost includes salary for two attendants and cost for leasing station space = $150,000

TIP cost: $200,000 +$150,000 x 3 = $650,000

Annualized cost:  $200,000/10 + $150,000  = $170,000

2005 Cost Effectiveness:

NOx:    		 		$170,000

250 days x 0 .0016 tons/day	   	=     $425,000/ ton

M-165   Bike Stations at Rail Station

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 107 |  | 115 |  |  |  |  |  |  |
| VMT | 1552 |  | 1573 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 115 | x | 0.9905 grs | x | 1 ton | = | 0.00013 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 1,573 | x | 0.6995 grs | x | 1 ton | = | 0.00121 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00134 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start+ Hot Soak | 115 | x | 2.3454 grs | x | 1 ton | = | 0.00030 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 1,573 | x | 0.2717 grs | x | 1 ton | = | 0.00047 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.00077 | tons |  |
| Cost effectiveness | $170,000 |  |  | = | 508274 | $/ton |  |  |  |
| NOx | 250 | x | 0.00134 |  |  |  |  |  |  |
| Cost effectiveness | $170,000 |  |  | = | 886146 | $/ton |  |  |  |
| VOC | 250 | x | 0.00077 |  |  |  |  |  |  |

| M –144 Parking Impact Fees |  |
| --- | --- |
| Description: | This measure would consist of a parking impact fee administered by local governments throughout the region. The fees would allow governments to recoup some of the costs associated with maintaining the roadway infrastructure and mitigating the adverse effects of automobile use. The fees could be charged per parking space to land owners annually or on a one-time basis. |
| Analysis Tool: | Sketch Planning , Mode Choice Model |

Assumptions:

- Two fee scenarios were evaluated:

- $250 annual fee applied to Montgomery County, MD, only.

- $250 annual fee applied throughout the metropolitan region.

- It was assumed the landowner would recoup the parking fees directly from the drivers parking on their property.

- The Metropolitan Washington Council of Governments’ (MWCOG) regional mode choice model was used to estimate the effects of increased parking fees on trip making behavior.  Using factors such as employment density and area type, the model forecasts average daily parking costs for each traffic analysis zone.  The proposed parking fees were converted to a daily cost of one dollar and added to the model’s forecasted parking cost.  The new costs (sum of forecasted costs plus proposed fees) were used to execute the mode choice model a second time.  The resulting trip forecasts indicate the effect of the proposed cost on the public’s travel mode choices.

- All cost calculations are in 2001 dollars.

- Life cycle of cost analysis is one year.

Summary of Impacts (2005)

|  | Alternative #1 (Montgomery Co.) | Alternative #2 (Regional) |
| --- | --- | --- |
| Daily VT Reduction | 16,950 VT | 118,276 VT |
| Daily VMT Reduction | 245,775 VMT | 1,715,002 VMT |
| Daily Nox Reductions | 0.258 tons | 1.794 tons |
| Daily VOC Reductions | 0.134 tons | 0.926 tons |

## Emission Analysis

2005 Trip Summary

| Scenario | Automobile Vehcile Trips |  |  | Transit Person Trips |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Trips | % Increase From Base | Trip Reduction from Base | Trips | % Increase from Base | Transit Trip Increase |
| Base Forecast | 3,615,788 | 0 | 0 | 669,825 | 0 | - |
| Alternative #1 Base plus $250 Annually (Montgomery County) | 3,598,838 | -0.47% | 16,950 | 679,279 | 1.41% | 9,451 |
| Alternative #2 Base plus $250 Annually (Regionally) | 3,497,512 | -3.27% | 118,276 | 747,007 | 11.52% | 77,182 |

Notes:

- The automobile vehicle trips include both single occupancy automobiles and high occupancy automobiles.

The automobile vehicle trip (VT) values in the table above can be used to calculate the reduction in vehicle miles of travel (VMT) for each alternative.

Alternative #1:  $250 Annual Fee applied to Montgomery County.

VTmc = 16,950

VMTmc = 16,950 x 14.5 = 245,775 VMT

Alternative #2: $250 Annual Fee applied Regionally

VTregion = 118,276

VMTregion = 118,276x 14.5 = 1,715,002 VMT

Daily Emission Savings (2005)

Daily NOx emissions reductions:

Alternative #1

Montgomery County Fees

| Cold Start | 16,950 trips | x | 0.912 grs | x | 1 ton | = | 0.017 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 trip |  | 907,185 grams |  |  |
| Running | 245,775 VMT | x | 0.980 grs | x | 1 ton | = | 0.266 tons |
|  |  |  | 1 mi |  | 907,185 grams |  |  |
|  |  |  |  |  | Total |  | 0.283 tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 0.258 tons |

Alternative #2

Regional Fees

| Cold Start | 118,276 trips | x | 0.912 grs | x | 1 ton | = | 0.119 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 trip |  | 907,185 grams |  |  |
| Running | 1,715,002 VMT | x | 0.980 grs | x | 1 ton | = | 1.853 tons |
|  |  |  | 1 mi |  | 907,185 grams |  |  |
|  |  |  |  |  | Total |  | 1.972 tons |
|  |  |  |  | Tier 2 Controls (0.91) |  |  | 1.794 tons |

Daily VOC emissions reductions:

Alternative #1

Montgomery County Fees

| Cold Start | 16,950 trips | x | 2.287 grs | x | 1 ton | = | 0.043 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
| + Hot Soak |  |  | 1 trip |  | 907,185 grams |  |  |
| Running | 245,550 VMT | x | 0.347 grs | x | 1 ton | = | 0.094 tons |
|  |  |  | 1 mi |  | 907,185 grams |  |  |
|  |  |  |  |  | Total |  | 0.137 tons |
|  |  |  |  | Tier 2 Controls (0.97) |  |  | 0.134 tons |

Alternative #2

Regional Fees

| Cold Start | 118,276 trips | x | 2.287 grs | x | 1 ton | = | 0.298 tons |
| --- | --- | --- | --- | --- | --- | --- | --- |
| + Hot Soak |  |  | 1 trip |  | 907,185 grams |  |  |
| Running | 1,715,002 VMT | x | 0.347 grs | x | 1 ton | = | 0.656 tons |
|  |  |  | 1 mi |  | 907,185 grams |  |  |
|  |  |  |  |  | Total |  | 0.954 tons |
|  |  |  |  | Tier 2 Controls (0.97) |  |  | 0.926 tons |

Cost Analysis

Revenue from TERM:

The number of parking spaces existing in 2005 was forecasted by assuming 100% of the base parking demand will be provided.

Applying the $250 annual fee to the parking spaces results in the total revenue from the spaces.

Alternative #1 Fees Applied in Montgomery County Only

| 475,487 | Trips | x | 1 space | x | $250 | = | $ 59,435,875/ year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 trips |  | year |  |  |

Alternative #2 Fees Applied Regionally

(For revenue estimation staff discounted trips that would be reduced by other TERMs already implemented in the region.  These estimates were based on off-line analysis and therefore were not part of the travel demand model)

VT reduction from other TERMs funded exclusively for emissions reduction by 2005 = 100,000

VT remaining for revenue estimation =  3,497,512 – 100,000 = 3,397,512

| 3,397,512 | Trips | x | 1 space | x | $250 | = | $ 424,468,900 / year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 trips |  | year |  |  |

Cost of implementing TERM:

The cost of increased transit service to accommodate the additional transit demand was calculated using generalized cost rates obtained from Washington Metropolitan Area Transit Authority (WMATA)*.

Alternative #1 Fees Applied in Montgomery County

| 9,454 | Trips/day | x | $0.24 (capital cost) | x | 250 days | = | $ 567,240 / year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip |  | year |  |  |

| 9,454 | Trips/day | x | $3.37 (operating cost) | x | 250 days | = | $ 7,964,995 / year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip |  | year |  |  |
| 9,454 | Trips/day | x | -$1.65 (fare revenue) Trip | x | 250 days year | = | -$ 3,876,140 / year |
|  |  |  |  |  | Total |  | $ 4,656,095 / year |

Net Revenue: $59,435,875 – $4,656,095 = $54,779,800/year

Alternative #2 Fees Applied in Regionally

| 77,182 | Trips/day | x | $0.24 (capital cost) | x | 250 days | = | $ 4,630,930 / year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip |  | year |  |  |

| 77,182 | Trips/day | x | $3.37 (operating cost) | x | 250 days | = | $ 65,025,835 / year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trip |  | year |  |  |
| 77,182 | Trips/day | x | -$1.65 (fare revenue) | x | 250 days | = | -$ 31,837 / year |
|  |  |  | Trip |  | year |  |  |
| Total |  |  |  |  |  |  | $37,819,180/year |

*Note:

- Transit cost rates obtained in 1993 from Washington Area Metropolitan Transit Authority (WMATA) were adjusted to 2001 dollars with a 3% annual inflation rate.

Net Revenue: $ 424,468,900 - $37,819,200 = $ 386,649,700/year

3 year revenue: = 3 x $ 386,649,700 = $ 1.16 billion

Cost Effectiveness (2005)

Alternative #1 Fees Applied in Montgomery County

| NOx | $ ( 54,779,800) | x | Day | x | year | = | $ 876,500 / ton |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | year |  | 0.258 ton |  | 250 days |  | (Revenue) |

| VOC | $ (54,779,800) | x | Day | x | year | = | $ 1,635,200 / ton |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | year |  | 0. 134 ton |  | 250 days |  | (Revenue) |

Alternative #2 Fees Applied Regionally

| NOx | $(386,649,700) | x | Day | x | year | = | $ 862,100 / ton |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | year |  | 1.794 tons |  | 250 days |  | (Revenue) |

| VOC | $ (386,649,700) | x | Day | x | year | = | $ 1,670,200 / ton |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | year |  | 0.926 tons |  | 250 days |  | (Revenue) |

M –144 Parking Impact Fees

| Revised Calculations using version 2.1 Travel demand Model and Mobile 6 emissions factors: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prior year analysis were revised using Mobile 6 and version 2.1 travel demand model. |  |  |  |  |  |  |  |  |  |
|  | VT |  | VMT |  |  |  |  |  |  |
| Version 1 | 18584517 |  | 164226427 |  |  |  |  |  |  |
| Version 2.1 | 19907288 |  | 166425325 |  |  |  |  |  |  |
| Correction factors | 1.071176 |  | 1.01338943 |  |  |  |  |  |  |
| Revised VT & VMT |  |  |  |  |  |  |  |  |  |
|  | Old |  | Revised |  |  |  |  |  |  |
| VT | 118276 |  | 126694 |  |  |  |  |  |  |
| VMT | 1715000 |  | 1737963 |  |  |  |  |  |  |
| Revised Nox Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start | 126694 | x | 0.9905 grs | x | 1 ton | = | 0.13833 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 1,737,963 | x | 0.6995 grs | x | 1 ton | = | 1.34009 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 1.47841 | tons |  |
| Revised VOC Estimation |  |  |  |  |  |  |  |  |  |
| Cold Start + Hot Soak | 126694 | x | 2.3454 grs | x | 1 ton | = | 0.32755 | tons |  |
|  |  |  | 1 mi |  | 907,185 grs |  |  |  |  |
| Running | 1,737,963 | x | 0.2717 grs | x | 1 ton | = | 0.52052 | tons |  |
|  |  |  | 1 mi |  | 907,185 |  |  |  |  |
|  |  |  |  |  | Total |  | 0.84807 | tons |  |
| Revenue/ton |  |  |  |  |  |  |  |  |  |
| Cost effectiveness | $386,649,700 |  |  | = | 838237 | $/ton |  |  |  |
| NOx | 312 | x | 1.47841 |  |  |  |  |  |  |
| Cost effectiveness | $386,649,700 |  |  | = | 1461278 | $/ton |  |  |  |
| VOC | 312 | x | 0.84807 |  |  |  |  |  |  |