###### Memorandum

Date:		December 5, 2005

To: 		Michael Clifford, COG/TPB

- From: 		Sunil Kumar, COG/MWAQC

- Joan Rohlfs, COG/MWAQC

Subject:	Mob6 inventories needed for 8-hour ozone SIP

The purpose of this memorandum is to inform COG/TPB staff of Mob6 inventories that will be needed for base, rate of further progress (RFP), and attainment years for 8-hour ozone SIP.

Based on the MWAQC staff meeting with EPA regarding the Phase-2 8-hour ozone SIP (dt. 11.22.2005), MWAQC staff has learned that these inventories will be needed as part of 8-hour ozone SIP. VOC, NOx, and CO emissions both on annual and typical summer weekday basis will be need to be submitted to EPA in NIF3.0 format by June 2006 for the base year 2002 as per new EPA emissions inventory guidance document (August 2005) and an EPA document provided in the above meeting by them. In addition, for RFP and modeling inventories emissions of VOC and NOx for a typical summer weekday only will be required.

While MWAQC staff will provide Mob6 emission factors for these runs, TPB staff will generate total emissions after applying these emission factors to the Travel Demand Model. TPB staff will also provide total emissions broken down by vehicle classes, HPMS facility types, and emission types for the base year 2002 only for the purpose of reporting these emissions to the EPA Region III in NIF3.0 format required by them by June 2006. Description of Mob6 runs are being provided below.

Description of Mobile6 Runs:

Run #1:	Base year 2002 Mob6 run (Actual Annual) for VOC, NOx, & CO*

- All 2002 control programs, 2002 Registration data/Diesel sales fraction/VMT mix, and 2002 total VMT

Run #2:	Base year 2002 Mob6 run (Typical summer weekday) for VOC, NOx, & CO*

- All 2002 control programs, 2002 Registration data/Diesel sales fraction/VMT mix, and 2002 total VMT

Run #3:	Adjusted year 2002 Mob6 run (For RFP; typical summer weekday) for VOC & NOx

- No post-1990 Clean Air Act measures, 1990 I&M programs, RVP set at 7.8,

2002 Registration data/Diesel sales fraction/VMT mix, and 2002 total VMT

Run #4:	Adjusted year 2008 Mob6 run (For RFP; typical summer weekday) for VOC & NOx

- No post-1990 Clean Air Act measures, 1990 I&M programs, RVP set at 7.8,

2002 Registration data/Diesel sales fraction/VMT mix, and 2002 total VMT,

Model year in Mob6 input file set at 2008

Run #5:	Target year 2008 Mob6 run (For RFP; typical summer weekday) for VOC & NOx

- All 2008 control programs, Registration data/Diesel sales fraction/VMT mix valid for 2008, and 2008 total VMT

Run #6:	Attainment year 2009 Mob6 run (Typical summer weekday) for VOC & NOx

- All 2009 control programs, Registration data/Diesel sales fraction/VMT mix valid for 2009, and 2009 total VMT

Based on the current schedule for 8-hour ozone SIP Planning, it appears that while 2002 mobile emissions inventory need to be finalized by February 2006, RFP and Attainment year inventories need to be completed by April 2006.