- Regional Traffic Signal System Inventory

| Agency | Signal Controller | Number of Signals | Detection | Controller Software | Signal System | Software/Hardware Upgrades Available for TSP? | Policy Established for TSP? | Implemented TSP | Demonstration Projects | Emergency Vehicle Preemption Equipment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| District of Columbia DOT | Type 170 | 1,300 | Loop Detection for signals | BI Tran | QuicNet 4 | Currently ongoing testing, similar to VDOT | Yes, very accepting of TSP and proactive planning | Underway with pilot project (10 signals) | Georgia Avenue | Few intersections, Pilot project will be first intersections in DC that can also do signal priority |
| Maryland SHA | Econolite NEMA TS2 | 500 (MC operates) ~400-450 (PGC) | Loop/ (Video rebuilds) Opticom (EV preemption) in PG County-35-40% | Econolite ASC /Aries | Closed Loop/Distributed | Econolite has ‘bolt-on’ TSP device. SHA to talk with Econolite in January 2005 | No, mobility is of paramount importance | MD Rte 2 Preemption Project (MTA dropped bus route, turned off system) | Previously completed MD Rte 2 |  |
| Montgomery County | Econolite and others | 500 (MC maintains all signals Countywide) | Loop | Econolite and others | Centralized: Commtracs (maintained by Siemens, which is second by second) |  | Loosely, but traffic is highest priority | Systemwide, but deactivated recently due to latency issues |  |  |
| Prince George’s County | Naztec NEMA TS2 | 155 owned by PGC, in addition to SHA signals | Loop/video (all new installations) | Naztec (various versions) | Naztec’s Streetwise Closed loop now/In process of installing fiber connecting to trip center (2-3 years) | Naztec has an untested version of TSP available in the local controller | No Monitoring development of new technologies. Revisit signal timing 1-2 years | No | None planned |  |
| City of Laurel, MD | Econolite | 30-40 (no additional to PGC that belong to the city) | Loop |  | Distributed |  | No | No |  | Opticom under contract (also an old audio preemption) |
| City of Bowie, MD |  | No signals maintained |  |  |  |  |  |  |  |  |
| Virginia DOT | Type 170 | ~1,100 | Opticom | BI Tran |  |  | Yes, very accepting of TSP and proactive planning |  |  |  |
| Arlington County | NEMA EPAC |  | Opticom | SCOOT ACTRA | Centralized |  | Yes, | Underway | Columbia Pike |  |
| Fairfax County | Type 170 | <50 (County Fire/Rescue only responsible for signals at fire stations) | Opticom | BI Tran | Closed Loop/Distributed |  | Yes, supportive and actively planning and implementing along Connector Bus corridors | Yes, but on VDOT operated signals only | Richmond Highway |  |
| Prince William County | Type 170 |  | Opticom (installed) | BI Tran |  |  |  |  |  |  |
| Loudon County | Type 170 |  | Opticom (approved) | BI Tran |  |  |  |  |  |  |
| City of Alexandria, VA | Eagle TS-2 | 234 (100% belong to city) | inductive loop, Video, and microwave | EPAC Controller Software | Distributed – Eagle Actra | Both the controller firmware and central system software have transit priority capabilities. However, some modifications to the code will probably need to be made to tailor the system to the regions needs | Yes, but would require a comprehensive feasibility study for implementation |  |  | Local preemption at the signal at the fire station, no planned upgrades |
| City of Fairfax, VA | Eagle EPAC 300 | 58 | Loop, some video | Eagle’s Monarc system | Central | ACTRA upgrade (underway) includes some TSP capability if additional equipment is added. | No specific policy. However most city intersections are operated with actuated side-street phasing so that any extra side-street time is automatically allocated back to the arterial streets (which improves arterial flow including transit buses on those arterials |  | No | Tomar pre-emption at 8 intersections (change is in-process to Opticom)Pre-emption expansion limited by realization that vehicular pre-emption or priority shifts the Eagle control system into a transition mode that results in intersections losing coordination for the pre-emption period. |
| Town of Leesburg, VA | LMD 9000 NEMA TS-1 | 34 (maintained by the town) | Loop Opticom (intstalled) | BI Tran Mats Software | Dial – up system? | Yes (specifics unknown) | Yes (specifics unknown) |  | No | 6 signals |
| City of Herndon | LMD 9200’s | 35 | Loop, camera | LMD software; version 7.3.7 | Closed loop | Contact Not sure | No | No |  |  |
| City of Manassas, VA | Type 170 | ~500 | Opticom | BI Tran |  |  |  |  |  |  |