Metropolitan Washington Regional

Intelligent Transportation Systems Architecture

Appendix B

Architecture Entity Descriptions

DRAFT for Committee Review

January 13, 2005

Arch. Entity Name	Entity Description

Alerting and Advisory 	This terminator represents the federal, state, and local alerting and advisory systems that

Systems	provide alerts, advisories, and other potential threat information that is relevant to surface

transportation systems.  This includes systems such as the Information Sharing and Analysis

Centers (ISACs), the National Infrastructure Protection Center (NIPC), the Homeland Security

Advisory System (HSAS), and other systems that provide intelligence about potential,

imminent, or actual attacks on the transportation infrastructure or its supporting information

systems.

This terminator also represents the early warning and emergency alert systems operated by

federal, state, county, and local agencies that provide advisories and alerts regarding all types

of emergencies including natural hazards (floods, hurricanes, tornados, earthquakes),

accidents (chemical spills, nuclear power plant emergencies) and other civil emergencies such

as child abduction alerts that impact transportation system operation and/or require immediate

public notification.  Note that weather related watches and warnings, such as those issued by

the National Hurricane Center, are provided by both this terminator and the Weather Service

terminator since many alerting and advisory systems and the National Weather Service both

provide severe weather and related hazards information.

The alerts and advisories that are provided by the systems represented by this terminator are

based on analysis of potential threat information that is collected from a variety of sources,

including information collected by ITS systems.  The bidirectional interface with this terminator

allows potential threat information that is collected by ITS systems to be provided to the

alerting and advisory systems to improve their ability to identify threats and provide useful

and timely information.

The types of information provided by this terminator include general assessments and incident

awareness information, advisories that identify potential threats or recommendations to

increase preparedness levels, alerts regarding imminent or in-progress emergencies, and

specific threat information such as visual imagery used for biometric image processing.

Archived Data 	The Archived Data Management Subsystem collects, archives, manages, and distributes data

Management 	generated from ITS sources for use in transportation administration, policy evaluation, safety,

planning, performance monitoring, program assessment, operations, and research

applications.  The data received is formatted and tagged with attributes that define the data

source, conditions under which it was collected, data transformations, and other information

(i.e. meta data) necessary to interpret the data.  The subsystem can fuse ITS generated data

with data from non-ITS sources and other archives to generate information products utilizing

data from multiple functional areas, modes, and jurisdictions.  The subsystem prepares data

products that can serve as inputs to federal, state, and local data reporting systems.  This

subsystem may be implemented in many different ways.  It may reside within an operational

center and provide focused access to a particular agency's data archives.  Alternatively, it

may operate as a distinct center that collects data from multiple agencies and sources and

provides a general data warehouse service for a region.

Archived Data User 	This terminator represents the systems users employ to access archived data.  The general

Systems	interface provided from this terminator allows a broad range of users (e.g. planners,

researchers, analysts, operators) and their systems (e.g. databases, models, analytical tools,

user interface devices) to acquire data and analyses results from the archive.

Arch. Entity Name 	Entity Description

Driver	This terminator represents the human entity that operates a licensed vehicle on the roadway.

Included are operators of private, Transit, Commercial, and Emergency vehicles where the

data being sent or received is not particular to the type of vehicle.  Thus this terminator

originates driver requests and receives driver information that reflects the interactions which

might be useful to all drivers, regardless of vehicle classification.  The Driver terminator is the

operator of the Basic Vehicle terminator.  Information and interactions which are unique to

drivers of a specific vehicle type (e.g., fleet interactions with transit, commercial, or

emergency vehicle drivers) are covered separately.

Emergency Management	The Emergency Management Subsystem represents public safety, emergency management,

and other allied agency systems that support incident management, disaster response and

evacuation, security monitoring, and other security and public safety-oriented ITS

applications. The subsystem includes the functions associated with fixed and mobile public

safety communications centers including public safety call taker and dispatch centers

operated by police (including transit police), fire, and emergency medical services.  It includes

the functions associated with Emergency Operations Centers that are activated at local,

regional, state, and federal levels for emergencies and the portable and transportable systems

that support Incident Command System operations at an incident.  This subsystem also

represents other allied systems including centers associated with towing and recovery,

freeway service patrols, HAZMAT response teams, and mayday service providers.

The subsystem manages sensor and surveillance equipment used to enhance transportation

security of the roadway infrastructure (including bridges, tunnels, interchanges, and other key

roadway segments) and the public transportation system (including transit vehicles, public

areas such as transit stops and stations, facilities such as transit yards, and transit

infrastructure such as rail, bridges, tunnels, or bus guideways). The subsystem provides

security/surveillance services to improve traveler security in public areas not a part of the

public transportation system.

This subsystem monitors alerts, advisories, and other threat information and prepares for and

responds to identified emergencies.  It interfaces with other Emergency Management

Subsystems to support coordinated emergency response involving multiple agencies. The

subsystem stores, coordinates, and utilizes emergency response and evacuation plans to

facilitate this coordinated response.  As the response progresses, situation information

including damage assessments, response status, evacuation information, and resource

information are shared to keep all allied agencies appraised of the response.  Interface with

the Transit Management Subsystem allows coordinated use of transit vehicles to facilitate

response to major emergencies and to support evacuation efforts.  The Emergency

Management Subsystem also provides a focal point for coordination of the emergency and

evacuation information that is provided to the traveling public, including wide-area alerts when

immediate public notification is warranted.

The subsystem tracks and manages emergency vehicle fleets using real-time road network

status and routing information from the other center subsystems to aide in selecting the

emergency vehicle(s) and routes that will provide the most timely response.  Interface with

the Traffic Management Subsystem allows strategic coordination in tailoring traffic control to

support emergency vehicle ingress and egress, implementation of special traffic restrictions

and closures, evacuation traffic control plans, and other special strategies that adapt the

Emergency 	This terminator represents the telecommunications systems that connect a caller with a Public

Telecommunications 	Safety Answering Point (PSAP).  These systems transparently support priority wireline and

System	wireless caller access to the PSAP through 9-1-1 and other access mechanisms like 7 digit

local access numbers, and motorist aid call boxes.   The calls are routed to the appropriate

PSAP, based on caller location when this information is available.  When available, the caller’s

location and call-back number are also provided to the PSAP by this interface.

Arch. Entity Name	Entity Description

Emergency Vehicle 	This subsystem resides in an emergency vehicle and provides the sensory, processing,

Subsystem	storage, and communications functions necessary to support safe and efficient incident

response. The subsystem represents a range of vehicles including those operated by police,

fire, and emergency medical services.  In addition, this subsystem represents other incident

response vehicles including towing and recovery vehicles and freeway service patrols.  The

Emergency Vehicle Subsystem includes two-way communications to support coordinated

response to emergencies in accordance with an associated Emergency Management

Subsystem.  Emergency vehicles are equipped with automated vehicle location capability for

monitoring by vehicle tracking and fleet management functions in the Emergency

Management Subsystem.  Using these capabilities, the appropriate emergency vehicle to

respond to each emergency is determined.  Route guidance capabilities within the vehicle

enable safe and efficient routing to the emergency.  In addition, the emergency vehicle may

be equipped to support signal preemption through communications with the Roadway

Subsystem.

Enforcement Agency	This terminator represents the systems that receive reports of violations detected by various

ITS facilities including individual vehicle emissions, toll violations, CVO violations, excessive

speed in work zones, etc.

Event Promoters	This terminator represents Special Event Sponsors that have knowledge of events that may

impact travel on roadways or other modal means.  Examples of special event sponsors

include sporting events, conventions, motorcades/parades, and public/political events.  These

promoters interface to the ITS to provide event information such as date, time, estimated

duration, location, and any other information pertinent to traffic movement in the surrounding

Information Service 	This subsystem collects, processes, stores, and disseminates transportation information to

Provider	system operators and the traveling public.  The subsystem can play several different roles in

an integrated ITS.  In one role, the ISP provides a general data warehousing function,

collecting information from transportation system operators and redistributing this information

to other system operators in the region and other ISPs.  In this information redistribution role,

the ISP provides a bridge between the various transportation systems that produce the

information and the other ISPs and their subscribers that use the information.   The second

role of an ISP is focused on delivery of traveler information to subscribers and the public at

large.  Information provided includes basic advisories, traffic and road conditions, transit

schedule information, yellow pages information, ridematching information, and parking

information. The subsystem also provides the capability to provide specific directions to

travelers by receiving origin and destination requests from travelers, generating route plans,

and returning the calculated plans to the users.  In addition to general route planning for

travelers, the ISP also supports specialized route planning for vehicle fleets.  In this third role,

the ISP function may be dedicated to, or even embedded within, the dispatch system.

Reservation services are also provided in advanced implementations.  The information is

provided to the traveler through the Personal Information Access Subsystem, Remote

Traveler Support Subsystem, and various Vehicle Subsystems through available

communications links.  Both basic one-way (broadcast) and personalized two-way information

provision are supported.   The subsystem provides the capability for an informational

infrastructure to connect providers and consumers, and gather the market information needed

to assist in the planning of service improvements and in maintenance of operations.

Arch. Entity Name	Entity Description

Maintenance and 	The Maintenance and Construction Management Subsystem monitors and manages roadway

Construction 	infrastructure construction and maintenance activities.  Representing both public agencies and

Management	 private contractors that provide these functions, this subsystem manages fleets of

maintenance, construction, or special service vehicles (e.g., snow and ice control equipment).

The subsystem receives a wide range of status information from these vehicles and

performs vehicle dispatch, routing, and resource management for the vehicle fleets and

associated equipment.  The subsystem participates in incident response by deploying

maintenance and construction resources to an incident scene, in coordination with other center

subsystems.  The subsystem manages equipment at the roadside, including environmental

sensors and automated systems that monitor and mitigate adverse road and surface weather

conditions.  The subsystem manages the repair and maintenance of both non-ITS and ITS

equipment including the traffic controllers, detectors, dynamic message signs, signals, and

other equipment associated with the roadway infrastructure.  Additional interfaces to weather

information providers (the weather service and surface transportation weather service

providers) provide current and forecast weather information that can be fused with other data

sources and used to support advanced decision support systems that increase the efficiency

and effectiveness of maintenance and construction operations.

The subsystem remotely monitors and manages ITS capabilities in work zones, gathering,

storing, and disseminating work zone information to other systems.  It manages traffic in the

vicinity of the work zone and advises drivers of work zone status (either directly at the

roadside or through an interface with the Information Service Provider or Traffic Management

subsystems.)    It schedules and manages the location and usage of maintenance assets

(such as portable dynamic message signs).

Construction and maintenance activities are tracked and coordinated with other systems,

improving the quality and accuracy of information available regarding closures and other

roadway construction and maintenance activities.

Media	This terminator represents the information systems that provide traffic reports, travel

conditions, and other transportation-related news services to the traveling public through radio,

TV, and other media.  Traffic and travel advisory information that are collected by ITS are

provided to this terminator.  It is also a source for traffic flow information, incident and special

event information, and other events which may have implications for the transportation

Multimodal 	This terminator provides the interface through which Transportation Service Providers can

Transportation Service 	exchange data with ITS. They are the operators of non-roadway transportation systems (e.g.

Provider	airlines, ferry services, passenger carrying heavy rail) and providers of non-motorized

transportation facilities. This two-way interface enables coordination for efficient movement of

people across multiple transportation modes. It also enables the traveler to efficiently plan

itineraries which include segments using modes not directly included in the ITS User Services.

Other Emergency 	Representing other Emergency Management centers, systems or subsystems, this terminator

Management	 provides a source and destination for ITS data flows between various communications

centers operated by public safety agencies, emergency management agencies, other allied

agencies, and private companies that participate in coordinated management of highway-

related incidents, including disasters.  The interface represented by this terminator enables

emergency management activities to be coordinated across jurisdictional boundaries and

between functional areas. In the Physical Architecture this terminator is a reciprocal

Emergency Management Subsystem (EM) implying the requirements for general networks

connecting many allied agencies.  The interface between this terminator and the EM supports

coordination of incident management information between many different centers providing

Public Safety Answering Point (both public or private sector implementations), Public Safety

Dispatch, Emergency Operations Centers, and other functions that participate in the detection,

verification, response, recovery and clearance of incidents, including disasters.  This

terminator also supports interface to other allied agencies like utility companies that also

participate in the coordinated response to selected highway-related incidents.

Arch. Entity Name	Entity Description

Other ISP	Representing other distinct Information Service Providers, this  terminator  is intended to

provide a source and destination for ITS data flows between peer  information and service

provider functions.   It enables cooperative information sharing between providers as

conditions warrant.  In the Physical Architecture this terminator is a reciprocal Information

Service Provider (ISP) Subsystem.

Other Traffic 	Representing another Traffic Management center, system or subsystem, this  terminator  is

Management	intended to provide a source and destination for ITS data flows between peer (e.g. inter-

regional) traffic management functions.   It enables traffic management activities to be

coordinated across different jurisdictional areas.  In the Physical Architecture, this terminator

is a reciprocal Traffic Management Subsystem (TMS).

Other Transit 	Representing another Transit Management center, system or subsystem, this terminator  is

Management	intended to provide a source and destination for ITS data flows between peer (e.g. inter-

regional) transit management functions.   It enables transit management activities to be

coordinated across geographic boundaries or different jurisdictional areas.  In the Physical

Architecture this terminator represents a reciprocal Transit Management Subsystem (TRMS).

Parking Management	The Parking Management Subsystem provides electronic monitoring and management of

parking facilities.  It supports a dedicated short-range communications (DSRC) link to the

Vehicle Subsystem that allows electronic collection of parking fees.  It also includes the

instrumentation, signs, and other infrastructure that monitors parking lot usage and provides

local information about parking availability and other general parking information.  This portion

of the subsystem functionality must be located in the parking facility where it can monitor,

classify, and share information with customers and their vehicles.  The subsystem also

interfaces with the financial infrastructure and broadly disseminates parking information to

other operational centers in the region.  Note that the latter functionality may be located in a

back office, remote from the parking facility.

Personal Information 	This subsystem provides the capability for travelers to receive formatted traffic advisories

Access	from their homes, place of work, major trip generation sites, personal portable devices, over

multiple types of electronic media.  These capabilities also provide basic routing information

and allow users to select those transportation modes that allow them to avoid congestion, or

more advanced capabilities to allow users to specify those transportation parameters that are

unique to their individual needs and receive travel information.  This subsystem provides

travelers with the capability to receive route planning from the infrastructure at fixed locations

such as in their homes, their place of work, and at mobile locations using personal portable

devices and vehicle-based devices.  In addition to end user devices, this subsystem may

also represent a device that is used by a merchant or other service provider to receive

traveler information and relay important information to their customers.  This subsystem also

provides the capability to initiate a distress signal and cancel a prior-issued manual request for

help.

Rail Operations	This terminator represents the (usually) centralized control point for a substantial segment of a

freight railroad's operations and maintenance activities. It is roughly the railroad equivalent to

a highway Traffic Management Center. It is the source and destination of information that can

be used to coordinate rail and highway traffic management and maintenance operations. It is

also the source and destination for incident, incident response, disaster, or evacuation

information that is exchanged with Emergency Management.   This terminator would also

represent a railroad's management information system, if that system is the source or

destination for this information. The use of a single terminator for multiple sources and

destination for information exchange with railroads  is meant to imply the need for a single,

consistent interface between a given railroad's operations and maintenance activities and ITS.

Arch. Entity Name	Entity Description

Remote Traveler 	This subsystem provides access to traveler information at transit stations, transit stops, other

Support	 fixed sites along travel routes (e.g., rest stops, merchant locations), and major trip generation

locations such as special event centers, hotels, office complexes, amusement parks, and

theaters.  Traveler information access points include kiosks and informational displays

supporting varied levels of interaction and information access.  At transit stops, simple

displays providing schedule information and imminent arrival signals can be provided. This

basic information may be extended to include multi-modal information including traffic

conditions and transit schedules along with yellow pages information to support mode and

route selection at major trip generation sites. Personalized route planning and route guidance

information can also be provided based on criteria supplied by the traveler.  The subsystem

also supports electronic payment of transit fares.

In addition to the traveler information provisions, this subsystem also supports security and

safety monitoring of public areas.  This monitoring includes traveler activated silent alarms, as

well as surveillance and sensor equipment.     The surveillance equipment includes video (e.g.

CCTV cameras) and/or audio systems. The sensor equipment includes threat sensors (e.g.

chemical agent, toxic industrial chemical, biological, explosives, and radiological sensors) and

object detection sensors (e.g. metal detectors).

Roadway Subsystem	This subsystem includes the equipment distributed on and along the roadway that monitors

and controls traffic and monitors and manages the roadway itself.  Equipment includes traffic

detectors, environmental sensors, traffic signals, highway advisory radios, dynamic message

signs, CCTV cameras and video image processing systems, grade crossing warning

systems, and freeway ramp metering systems.  HOV lane management, reversible lane

management functions, and barrier systems that control access to transportation

infrastructure such as roadways, bridges and tunnels are also supported.  This subsystem

also provides the capability for environmental monitoring including sensors that measure road

conditions, surface weather, and vehicle emissions.  In adverse conditions, automated

systems can be used to apply anti-icing materials, disperse fog, etc.  Work zone systems

including work zone surveillance, traffic control, driver warning, and work crew safety systems

are also included.   To enhance security, safeguard systems such as blast shields, exhaust

systems and other automated and remotely controlled systems to protect transportation

infrastructure is also provided. In advanced implementations, this subsystem supports

automated vehicle safety systems by safely controlling access to and egress from an

Automated Highway System through monitoring of, and communications with, AHS vehicles.

Intersection collision avoidance functions are provided by determining the probability of a

collision in the intersection and sending appropriate warnings and/or control actions to the

approaching vehicles.

Telecommunications 	This terminator provides the caller interface and voice processing (voice

System for Traveler 	recognition/synthesis) that supports voice-enabled traveler telephone information systems.   It

Information	 provides wireline and wireless caller access to 511 systems and other telephone access

mechanisms like 7 or 10 digit local access numbers.  It represents the boundary of the

architecture where a call is received and processed and includes voice portal capabilities in

scenarios where a distinct voice portal exists between ITS Centers and telecommunications

providers.  The terminator gathers traveler information, alerts, and advisories from information

service provider(s) and uses this information to support voice-based interactions with a

Toll Administration	The Toll Administration Subsystem provides general payment administration capabilities and

supports the electronic transfer of authenticated funds from the customer to the transportation

system operator.  This subsystem supports traveler enrollment and collection of both pre-

payment and post-payment transportation fees in coordination with the existing, and evolving

financial infrastructure supporting electronic payment transactions.  The system may

establish and administer escrow accounts depending on the clearinghouse scheme and the

type of payments involved.  This subsystem posts a transaction to the customer account and

generates a bill (for post-payment accounts), debits an escrow account, or interfaces to the

financial infrastructure to debit a customer designated account.   It supports communications

with the Toll Collection Subsystem to support fee collection operations.  The subsystem also

sets and administers the pricing structures and includes the capability to implement road

pricing policies in coordination with the Traffic Management Subsystem.  The electronic

financial transactions in which this subsystem is an intermediary between the customer and

the financial infrastructure shall be cryptographically protected and authenticated to preserve

privacy and ensure authenticity and auditability.

Arch. Entity Name	Entity Description

Toll Collection	The Toll Collection Subsystem provides the capability for vehicle operators to pay tolls without

stopping their vehicles using locally determined pricing structures and includes the capability

to implement various variable road pricing policies.  Each transaction is accompanied by

feedback to the customer indicating the general status of the customer account.  A record of

the transactions is provided to the Toll Administration Subsystem for reconciliation and so that

the customer can periodically receive a detailed record of the transactions.

Traffic Management	The Traffic Management Subsystem monitors and controls traffic and the road network.  It

represents centers that manage a broad range of transportation facilities including freeway

systems, rural and suburban highway systems, and urban and suburban traffic control

systems.  This subsystem communicates with the Roadway Subsystem to monitor and

manage traffic flow and monitor the condition of the roadway, surrounding environmental

conditions, and field equipment status.  This subsystem coordinates with the Maintenance and

Construction Management Subsystem to maintain the road network and coordinate and adapt

to maintenance activities, closures, and detours.  Incidents are detected, verified, and incident

information is provided to allied agencies, drivers (through Roadway Subsystem highway

advisory radio and dynamic message signs), and information service providers.  This

subsystem also manages traffic and transportation resources to support allied agencies in

responding to, and recovering from, incidents ranging from minor traffic incidents through

major disasters.  When required, special traffic management strategies are implemented to

support evacuation and reentry.  The Traffic Management Subsystem supports HOV lane

management and coordination, road pricing, and other demand management policies that can

alleviate congestion and influence mode selection.   It also manages reversible lane facilities

and barrier and safeguard systems that control access to transportation infrastructure. The

subsystem communicates with other Traffic Management Subsystems to coordinate traffic

information and control strategies in neighboring jurisdictions. It also coordinates with rail

operations to support safer and more efficient highway traffic management at highway-rail

intersections.  Finally, the Traffic Management Subsystem provides the capabilities to

exercise control over those devices utilized for automated highway system (AHS) traffic and

vehicle control.

Transit Management	The Transit Management Subsystem manages transit vehicle fleets and coordinates with other

modes and transportation services.  It provides operations, maintenance, customer

information, planning and management functions for the transit property.  It spans distinct

central dispatch and garage management systems and supports the spectrum of fixed route,

flexible route, paratransit services, transit rail, and bus rapid transit (BRT) service.  The

subsystem's interfaces allow for communication between transit departments and with other

operating entities such as emergency response services and traffic management systems.

This subsystem receives special event and real-time incident data from the traffic

management subsystem.   It provides current transit operations data to other center

subsystems.  It interfaces with the Emergency Management Subsystem to allow coordinated

use of transit vehicles to facilitate response to major emergencies or evacuations. The Transit

Management Subsystem collects and stores accurate ridership levels and implements

corresponding fare structures. It collects operational and maintenance data from transit

vehicles, manages vehicle service histories, and assigns vehicle operators and maintenance

personnel to vehicles and routes.  The Transit Management Subsystem also provides the

capability for automated planning and scheduling of public transit operations.  It furnishes

travelers with real-time travel information, continuously updated schedules, schedule

adherence information, transfer options, and transit routes and fares.  In addition, the

subsystem supports transit security features.  This includes monitoring silent alarms, both

passenger and operator initiated, on-board transit vehicles.   It also includes the capability to

support transit vehicle operator authentication and the capability to remotely disable a transit

vehicle.  The subsystem includes the capability to monitor for a transit vehicle being off the

assigned route.   The subsystem also includes the capability to alert operators and police to

potential incidents identified by these security features.

Arch. Entity Name	Entity Description

Transit Vehicle 	This subsystem resides in a transit vehicle and provides the sensory, processing, storage,

Subsystem	and communications functions necessary to support safe and efficient movement of

passengers. The types of transit vehicles containing this subsystem include buses,

paratransit vehicles, light rail vehicles, other vehicles designed to carry passengers, and

supervisory vehicles.   The subsystem collects accurate ridership levels and supports

electronic fare collection.  The subsystem supports a traffic signal prioritization function that

communicates with the roadside subsystem to improve on-schedule performance.  Automated

vehicle location functions enhance the information available to the Transit Management

Subsystem enabling more efficient operations.  On-board sensors support transit vehicle

maintenance.  The subsystem supports on-board security and safety monitoring.  This

monitoring includes transit user or vehicle operator activated alarms (silent or audible), as well

as surveillance and sensor equipment.     The surveillance equipment includes video (e.g.

CCTV cameras), audio systems and/or event recorder systems. The sensor equipment

includes threat sensors (e.g. chemical agent, toxic industrial chemical, biological, explosives,

and radiological sensors) and object detection sensors (e.g. metal detectors).  In addition, the

subsystem supports vehicle operator authentication prior to operation of the vehicle and

remote vehicle disabling.  The subsystem also furnishes travelers with real-time travel

information, continuously updated schedules, transfer options, routes, and fares.

Traveler	This terminator represents any individual who uses transportation services.  The interfaces to

the traveler provide general pre-trip and en-route information supporting trip planning, personal

guidance, and requests for assistance in an emergency that are relevant to all transportation

system users.  . The terminator represents users of a public transportation system and

addresses interfaces these users have within a transit vehicle or at transit facilities such as

roadside stops and transit centers. This general terminator is supplemented in the architecture

by the specific "Driver" terminator that supports interfaces that are specific to drivers.

Traveler Card	This terminator represents the entity that enables the actual transfer of electronic information

from the user of a service (I.e. a traveler) to the provider of the service.  This may include

the transfer of funds through means of an electronic payment instrument. The device, like a

smart card, may also hold and update the traveler's information such as personal profiles.