Effectiveness Estimates for Urban BMPs

n represents the number of studies

Urban wetlands and wet ponds

Table 1. Summary of pollutant removal effectiveness (%) for stormwater wetlands and wet ponds evaluated in single-site studies.

| Statistic | TSS (Total Suspended Solids) | TN (Total Nitrogen) | TP (Total Phosphorus) |
| --- | --- | --- | --- |
| Mean | 56 | 17 | 45 |
| Standard deviation | 36.5 | 29.3 | 25.7 |
| Standard error | 4.5 | 4.5 | 3.3 |
| Median | 64 | 20 | 45 |
| Minimum | -78 | -81 | -42 |
| Maximum | 99 | 65 | 86 |
| n (UMD project review) | 62 | 40 | 58 |
| n (current effectiveness estimate) | 4 | 3 | 5 |
| Effectiveness Estimate Recommendation: |  |  |  |
| Current | 80 | 30 | 50 |
| Developer | 60 | 20 | 45 |
| Initial UMD/MAWP recommendation | 50 | 15 | 40 |
| New CBP Effectiveness Estimate | 60 | 20 | 45 |

Urban Erosion and Sediment Control

Table 2. Summary of literature on the pollutant removal effectiveness (%) of single-site studies and multi-site studies of urban erosion and sediment controls as Best Management Practices for urban and mixed open land uses.

| Statistic | TSS | TN | TP |
| --- | --- | --- | --- |
| Mean | 74 |  |  |
| Minimum | 0 |  |  |
| Maximum | 99 |  |  |
| n (UMD project review) | 23 | 0 | 1* |
| n (current effectiveness estimates) | 0 (best professional judgment) | 0 (best professional judgment) | 0 (best professional judgment) |
| Effectiveness Estimation Recommendation |  |  |  |
| Current | 50 | 30 | 50 |
| Developer | Current effectiveness estimate too high – but data not sufficient to recommend efficiency | No info available to recommend effectiveness estimate | Current estimate too high – but data not sufficient to recommend effectiveness estimate |
| Initial UMD/MAWP recommendation and New CBP Effectiveness Estimate | 40 | 25 | 40 |

* The value from this one study is a 20% reduction.

Dry extended detention basins

Table 3. Summary of literature on the pollutant removal effectiveness (%) of single-site studies and multi-site studies of dry extended detention basins Best Management Practices for urban and mixed open land uses.

| Statistic | TSS | TN | TP |
| --- | --- | --- | --- |
| Mean | 60 | 25 | 30 |
| Minimum | 30 | 15 | 15 |
| Maximum | 85 | 34 | 57 |
| n (UMD project review) | 5 | 5 | 5 |
| n (current effectiveness estimate) | 1* | 0 | 0 |
| Effectiveness Estimation Recommendation |  |  |  |
| Current | 60 | 30 | 20 |
| # Initial UMD/MAWP Recommendation and New CBP Effectiveness Estimate | 60 | 20 | 20 |

* data based on fewer than 5 data points

Dry detention ponds/basins and hydrodynamic structures

Table 4. Summary of literature on the pollutant removal effectiveness (%) of single-site studies and multi-site studies for dry detention basins and hydrodynamic structures.

| Statistic | TSS (Total Suspended Solids) | TN (Total Nitrogen) | TP (Total Phosphorus) |
| --- | --- | --- | --- |
| Average | 54 | 13 | 43 |
| Standard Deviation | 47.6 | 31.2 | 27.8 |
| Standard Error | 10.9 | 12.7 | 7.2 |
| Median | 75 | 18 | 38 |
| Minimum | -52 | -30 | -3 |
| Maximum | 98 | 44 | 88 |
| n (UMD project review) | 9 | 6 | 15 |
| n (current) | 1 | 1 | 1 |
| Recommended Effectiveness Estimate |  |  |  |
| Current | 10 | 5 | 10 |
| Developer and Initial UMD/MAWP Recommendation | 50 | 15 | 35 |
| New CBP Effectiveness Estimate | 10 | 5 | 10 |