BLUE PLAINS WWTP’s

OPERATIONAL PRETREATMENT REQUIREMENTS

(preliminary list & language, 4/7/06 COG staff draft))

Note:  This list represents items discussed with the IMA Negotiation Team to-date and is not to be considered complete or all inclusive.  This list simply serves to provide a framework for an initial BPTC discussion of these types of requirements and how best to ensure protection of Blue Plains’ processes and equipment.  This list does not include existing Pretreatment Program requirements that are driven by formal EPA requirements and/or Local Limits.

Ref. Key Principle #9 – DC-WASA is the permit holder for Blue Plains and is responsible for the management, operation, and maintenance of the facility.

OBJECTIVE

To establish operational pretreatment requirements for wastewater coming into Blue Plains as necessary to preclude damage to Blue Plains’ wastewater and biosolids processes and equipment, and therefore to preclude any negative impacts at the plant.

REQUIREMENTS

- Screening of incoming wastewater – Wastewater should receive coarse screening before reaching Blue Plains in order to protect the plant’s fine screens and filters.  Screenings [from upstream locations/pump stations, etc.] should not be ground and returned to [the plant’s influent] wastewater flow.

- Discharge of sludge or other residuals – No sludge or other residuals should be discharged [into any manholes, pipelines, or other points that would result in them being sent] to Blue Plains.

- Discharge of trucked waste from outside the BPSA – Trucked waste [generated] from outside the BPSA cannot be discharged into sewers tributary to Blue Plains.

- Discharge of trucked waste from inside the BPSA – [Authorized] trucked waste [generated] within the BPSA should only be discharged at monitored locations.  That monitoring must record both the volume and characteristics of the waste.

- Pretreatment Program requirements – Timely reporting and responsiveness to EPA’s requirements [are required of all entities that discharge wastewater to Blue Plains.]

Note:  New or modified submittal and response requirements and their enforcement may need to be developed to ensure these requirements are met.

input.doc